Connecticut Western Railroad

1869 – 1881

Locomotive Roster Notes

| NBR | Name | Builder | Const # | Build Date | Delivery Date | Cylinder Size | Driver Diam | Wheel Arrangement | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (none) | Falls Village | Rogers, Ketch. & Grosvenor | 435 | 10/31/1853 | 6/1871 | 13" x 20" | 60" | 4-4-0 | CW 1 |
| 1 | State Line | Rogers L&M Co. | *1869 | 3/20/1871 | 9/15/1871 | 15" x 22" | 48 3/4" | 2-4-4T | CW 2 |
| 2 | City of Hartford | Rogers L&M Co. | *1947 | 9/20/1871 | 10/12/1871 | 15" x 22" | 60" | 4-4-0 | CW 3 |
| 3 | Winsted | Rogers L&M Co. | *1955 | 10/03/1871 | 10/12/1871 | 16" x 24" | 60" | 4-4-0 | CW 4 |
| 4 | Salisbury | Rogers L&M Co. | *1968 | 10/28/1871 | 11/6/1871 | 15" x 22" | 60" | 4-4-0 | CW 5 |
| 5 | Norfolk | Rogers L&M Co. | *1969 | 10/30/1871 | 11/1871 | 16" x 24" | 60" | 4-4-0 | CW 6 |
| 6 | Bloomfield | Rogers L&M Co. | *1838 |  | 1/9/1871 | 16" x 22" | 66" | 4-4-0 | CW 7 |
| 7 | Canton | Rogers L&M Co. | *2079 | 6/7/1872 |  | 17" x 22" | 54" | 4-4-0 | CW 8 |
| 8 | Canaan | Rogers L&M Co. | *2080 | 6/10/1872 |  | 17" x 22" | 54" | 4-4-0 | CW 9 |
| 9 | Simsbury | Rogers L&M Co. | 2363 | 11/17/1873 |  | 17" x 22" | 54" | 4-4-0 | CW 10 |
| 10 | Tariffville | Rogers L&M Co. | 2429 |  | 12/10/1871 | 17" x 22" | 60" | 4-4-0 | CW 11 |
| 11 | Lakeville | Rogers L&M Co. | 2433 | 12/29/1875 |  | 15" x 22" | 60" | 4-4-0 | CW 12 |

(*) Indicates unofficial construction number as Rogers apparently used none between Jun 1856 and Nov 1872. (R&LHS Bulletin #167 – Fall, 1992)

CW 1

Bought in June 1871 from the Housatonic RR and used in the construction of the CW RR. It had copper tubes and blew up in Winsted, CT on 8 Mar 1872. It was repaired and sold to the Rhinebeck & Connecticut RR on 29 May 1872 for $1500.00 (for some reason w/o the tender). Never was assigned a road number by the CW RR. (See R&C note 1)

CW 2

Was delivered on 15 Sep 1871 and used in the construction of the CW RR. Had a headlight at both ends and used mainly as a switcher in Hartford, CT. Was shopped at Hartford in Sep 1890. Reportedly shipped by flatcar from storage in Canaan, CT to the Reading, PA shops in mid 1893 for scrapping. (to H&CW #1; CNE&W #1; PR&NE (1st) #1)

CW 3

Cost new $10,250 and was delivered to the CW RR on 12 Oct 1871. Was sold to the Shepaug RR on 20 Nov 1888 for between $2500.00 and $3000.00 and became their #4. Later became NH #554. The #2 spot vacancy was filled in Jan 1892 by the CNE&W with ex-R&C RR 0-4-2T #3 / (2nd) #2 that carried the #12 on the CNE&W prior to Jan 1892. (To H&CW #2)

CW 4

Delivered on 12 Oct 1871. Was shopped in Hartford, CT in Oct, 1890. Was taken to the Reading, PA shops in fall of 1893 for major repairs that were never made. Written off by the PR&NE RR in 4th quarter of 1895 and presumably scrapped by the P&R Railway soon after. (To H&CW #3; CNE&W #3; PR&NE #3)

CW 5

Delivered new at Canaan, CT on 6 Nov 1871 by Engineer John Barton. Was removed from road service in late 1892 and used as a stationary boiler for passenger car heating in Hartford, CT. Was shipped to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. on 18 Mar 1899 for scrapping. Number was replaced by new BLW 4-4-0 (2nd) #4 built in Dec 1893. (To H&CW #4; CNE&W #4; PR&NE (1st) #4)

CW 6

Delivered new in mid Nov 1871 at Canaan, CT. Was wrecked at Modena, NY on 13 Nov 1892. Was used on Poughkeepsie Bridge trolley shuttle service in Mar 1899 while regular loco was being repaired. Was out of service on the CNE Railway takeover of the PR&NE in Jan 1899 but was repaired and returned to service. Was sold to the American Iron & Steel Mfg. Co. and sent to Lebanon, PA for scrapping on 16 Jan 1901. (To H&CW #5; CNE&W #5; PR&NE  #5 - #1254 - #5; CNE (1st) #5)

CW 7

Built for the New Haven, Middletown, & Willimantic RR (#3 “Portland”) which defaulted on payment and was sold at auction by Rogers Locomotive Works to the CW RR for $10,250.00. Was delivered new at Canaan, CT in Sep 1871.Was knocked off the track and down the bank by a five (5) ton boulder at Stoney Lonesome (near West Norfolk, CT) on 8 Mar 1882 and it took ten (10) days to be rerailed. Was condemned on 9 Mar 1899 and sent to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. on 18 Mar 1899 for scrapping. (To H&CW #6; CNE&W #6; PR&NE  #6 - #1255 - #6; CNE (1st) #6)

CW 8

Ordered by the CW RR on 12 Apr 1872. Was scrapped at the Reading, PA shops in late 1893. The number was replaced by new BLW 4-4-0 (2nd) #7 built in Dec 1893. (To H&CW #7; CNE&W #7; PR&NE (1st) #7)

CW 9

Ordered by the CW RR on 12 Apr 1872. Was scrapped at the Reading, PA shops in early 1893. The number was replaced by new BLW 4-4-0 (2nd) #8 built in Dec 1893. (To H&CW #8; CNE&W #8; PR&NE (1st) #8)

CW 10

Cost new $14,500.00. Was taken to the Reading, PA shops in the fall of 1893 for major repairs that were never made.. Was written off by the PR&NE in the 4th quarter of 1895 and presumable scrapped by the P&R Railway soon after. (To H&CW #9; CNE&W #9; PR&NE #9)

CW 11

Ordered in Oct 1875 by the CW RR and was delivered on 10 Dec 1875. Cost new $9,000.00. Was wrecked at the Farmington River bridge west of Tariffville, CT on 15 Jan 1878. Was returned to service on 16 Feb 1878. Reportedly was out of service for four years from late 1893 until late 1897. Rebuilt at the Hartford, CT shops in Oct 1890, Jun 1898 and in Feb 1902. Was sold on 2 Nov 1906 to an I.U. Harris (scrap dealer) of Hartford. CT for $625.00. (To H&CW #10; CNE&W #10; PR&NE #10; CNE (1st) #10 - #210)

CW 12

Delivered new in Jan 1876. Was found to be too heavy for the CW RR’s light rails and was sold to the Housatonic RR on 12 Feb 1879 for $7000.00. Became HRR #20 “A.B. Mygatt”. Became NYNH&H RR #470 in 1892 and #1953 in 1904. Was scrapped in Jan 1905.

Hartford & Connecticut Western Railroad

1881 – 1889

Locomotive Roster Notes

| NBR | Name | Builder | Const # | Build Date | Delivery Date | Cylinder Size | Driver Diam | Wheel Arrangement | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | State Line | Rogers L&M Co. | *1869 | 3/20/1871 | 9/15/1871 | 15" x 22" | 48 3/4" | 2-4-4T | HCW 1 |
| 2 | City of Hartford | Rogers L&M Co. | *1947 | 9/20/1871 | 10/12/1871 | 15" x 22" | 60" | 4-4-0 | HCW 2 |
| 3 | Winsted | Rogers L&M Co. | *1955 | 10/03/1871 | 10/12/1871 | 16" x 24" | 60" | 4-4-0 | HCW 3 |
| 4 | Salisbury | Rogers L&M Co. | *1968 | 10/28/1871 | 11/6/1871 | 15" x 22" | 60" | 4-4-0 | HCW 4 |
| 5 | Norfolk | Rogers L&M Co. | *1969 | 10/30/1871 | 11/1871 | 16" x 24" | 60" | 4-4-0 | HCW 5 |
| 6 | Bloomfield | Rogers L&M Co. | *1838 |  | 1/9/1871 | 16" x 22" | 66" | 4-4-0 | HCW 6 |
| 7 | Canton | Rogers L&M Co. | *2079 | 6/7/1872 |  | 17" x 22" | 54" | 4-4-0 | HCW 7 |
| 8 | Canaan | Rogers L&M Co. | *2080 | 6/10/1872 |  | 17" x 22" | 54" | 4-4-0 | HCW 8 |
| 9 | Simsbury | Rogers L&M Co. | 2363 | 11/17/1873 |  | 17" x 22" | 54" | 4-4-0 | HCW 9 |
| 10 | Tariffville | Rogers L&M Co. | 2429 |  | 12/10/1871 | 17" x 22" | 60" | 4-4-0 | HCW 10 |
| 11 |  | Danforth L&M | 943 | 8/18/1873 |  | 17" x 24" | 60" | 4-4-0 | HCW 11 |
| 12 |  | Danforth L&M | 960 | 9/16/1873 |  | 16" x 24" | 46" | 0-4-2T | HCW 12 |
| 13 |  | Danforth L&M | 974 | 11/12/1873 |  | 17" x 24" | 60" | 4-4-0 | HCW 13 |
| 14 |  | Danforth L&M | 1017 | 8/18/1874 |  | 17" x 24" | 60" | 4-4-0 | HCW 14 |
| 15 |  | Rogers L&M Co. | 3052 | 8/7/1882 |  | 18" x 24" | 60" | 4-4-0 | HCW 15 |
| 16 |  | Rogers L&M Co. | 3096 | 9/30/1882 |  | 18" x 24" | 62" | 4-4-0 | HCW 16 |
| 17 |  | Rogers L&M Co. | 3299 | 7/6/1883 |  | 18" x 24" | 62" | 4-4-0 | HCW 17 |
| 18 |  | Rogers L&M Co. | 3346 | 9/10/1883 |  | 18" x 24" | 62" | 4-4-0 | HCW 18 |
| 19 |  | Rogers L&M Co. | 4108 | 2/11/1889 |  | 19" x 24" | 56" | 2-6-0 | HCW 19 |

(*) Indicates unofficial construction number as Rogers apparently used none between Jun 1856 and Nov 1872. (R&LHS Bulletin #167 – Fall, 1992)

HCW 1

Had a headlight at both ends and used mainly as a switcher in Hartford, CT. Was shopped at Hartford in Sep 1890. Reportedly shipped by flatcar from storage in Canaan, CT to the Reading, PA shops in mid 1893 for scrapping. (See CW Note 02) (Was CW #1; To CNE&W #1; PR&NE (1st) #1)

HCW 2

Was sold to the Shepaug RR on 20 Nov 1888 for between $2500.00 and $3000.00 where it became their #4. Became NH RR #554 in 1898. The #2 spot remained vacant until filled in Jan 1892 by the CNE&W with ex-R&C RR 0-4-2T #3 / (2nd) #2 that was CNE&W #12 prior to that date. (See CW RR Note 3) (Was CW #2)

HCW 3

Was shopped in Hartford, CT in Oct, 1890. Was taken to the Reading, PA shops in fall of 1893 for major repairs that were never made. Written off by the PR&NE RR in 4th quarter of 1895 and presumably scrapped by the P&R Railway soon after. (See CW RR Note 3) (Was CW #3; To CNE&W #3; PR&NE #3)

HCW 4

Leased to the Hudson Connecting RR in early 1889 for $216.00 per month and used on construction train between Highland and Maybrook; Was removed from road service in late 1892 due to poor performance and used as a stationary boiler at Hartford, CT for passenger cars. Was sold and shipped to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. on 18 Mar 1899 for scrapping. The vacant #4 spot was filled by the PR&NE RR in Dec 1893 by new 4-6-0 BLW (2nd) #4. (See CW RR Note 5) (Was #4; To CNE&W #4; PR&NE (1st) #4)

HCW 5
Was wrecked at Modena, NY on 13 Nov 1892. Was used on Poughkeepsie Bridge trolley shuttle service in Mar 1899 while regular loco was being repaired. Was out of service on the CNE Railway takeover of the PR&NE in Jan 1899 but was repaired and returned to service. Was sold to the American Iron & Steel Mfg. Co. and sent to Lebanon, PA for scrapping on 16 Jan 1901. (See CW RR Note 6) (Was CW #5; To CNE&W #5; PR&NE #5 - #1254 - #5; CNE (1st) #5)

HCW 6

Was knocked off the track and down the bank by a five (5) ton boulder at Stoney Lonesome (near West Norfolk, CT) on 8 Mar 1882 and it took ten (10) days for the locomotive to be brought back up to track level. Was condemned by the CNE on 9 Mar 1899 and sent to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. on 18 Mar 1899 for scrapping. (See CW RR Note 7) (Was CW #6; To CNE&W #6; PR&NE #6 - #1255 - #6; CNE (1st) #6)

HCW 7

Leased to the Hudson Connecting RR in early 1889 for $371.00 per month and used on construction train between Highland and Maybrook. Was scrapped by the P&R Railway at their Reading, PA shops in late 1893. The vacant #7 spot filled by the PR&NE in Dec 1893 by new BLW (2nd) #7. (See CW RR Note 8) (Was CW #7; To CNE&W #7; PR&NE (1st) #7)

HCW 8

Was scrapped by the P&R Railway at their Reading, PA shops in late 1893. The vacant #8 spot filled by the PR&NE in Dec 1893 by new BLW (2nd) #8. (See CW RR Note 9) (Was CW #8; To CNE&W #8; PR&NE (1st) #8)

HCW 9

Was taken to the P&R Railway’s Reading,PA shops in the fall of 1893 for repairs that were never made. Was written off by the PR&NE in the 4th quarter of 1895 and was scrapped by the P&R Railway soon after. (See CW RR Note 10) (Was CW #9; To CNE&W #9; PR&NE #9)

HCW 10

Reportedly was out of service on the PR&NE for four years from late 1893 until late 1897. Rebuilt at the Hartford, CT shops in Oct 1890, Jun 1898 and in Feb 1902. Was sold in Sep 1906 to an I.U. Harris (scrap dealer) of Hartford, CT for $625.00. (See CW RR Note 11) (Was CW #10; To CNE&W #10; PR&NE #10; CNE (1st) #10 - #210)

HCW 11

Was rebuilt from a hard coal to a soft coal burner by the H&CW RR soon after the takeover of the R&C RR in 1882. Renumbered by the CNE&W RR to #41 when new BLW 4-6-0 (2nd) #11 was delivered in Jan 1892. Was probably scrapped by the P&R Railway shortly after it’s takeover of the CNE&W RR in Aug, 1892. (See R&C Note 3) (Was R&C (1st) #2 – (2nd) #1; To CNE&W (1st) #11 - #41)

HCW 12

Was rebuilt from a hard coal to a soft coal burner by the H&CW RR soon after the takeover of the R&C RR in 1882. Used by the HC&W RR on the docks at Rhinecliff, NY and by the CNE&W RR and the PR&NE RR (after the Poughkeepsie Bridge opened) as a switcher in both Maybrook, NY and Hartford, CT. Was Renumbered by the CNE&W RR from (1st) #12 to #2 in Jan 1892 after delivery of new BLW 4-6-0 (2nd) #12. Was in a condemned state in Hartford, CT in Nov 1894 but was repaired and returned to service again as a switcher there. Was again in a condemned state upon the CNE Railway’s takeover of the PR&NE RR on 23 Jan 1899 and probably was scrapped soon after. (See R&C Note 4) (Was R&C (1st) #3 – (2nd) #2; To CNE&W (1st) #12 - #2; PR&NE #2 - #1251 - #2)

HCW 13

Was rebuilt from a hard coal to a soft coal burner by the H&CW RR soon after the takeover of the R&C RR in 1882. Renumbered by the CNE&W RR to #43 when new BLW 4-6-0 (2nd) #13 was delivered in Jan 1892. Was probably scrapped by the P&R Railway shortly after it’s takeover of the CNE&W RR in Aug, 1892. (See R&C Note 5) (Was R&C (1st) #4 – (2nd) #3; To CNE&W (1st) #13 - #43)

HCW 14

Was rebuilt from a hard coal to a soft coal burner by the H&CW RR soon after the takeover of the R&C RR in 1882. Was lost in the roundhouse fire at Rhinecliff, NY on 16 Dec 1887. The #14 spot was vacant until Jan 1892 when new CNE&W RR BLW 4-6-0 #14 was delivered. (See R&C Note 6) (Was R&C #5 – (2nd) #4)

HCW 15

Was built for the East Tennessee, Virginia & Georgia RR but diverted to the H&CWRR by the Rogers Locomotive Works. Cost new $13,224 and was used primarily on the Rhinecliff Branch due to it’s light weight. Was rebuilt in spring 1892 by Rogers. Was condemned in Apr 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $200.00. (To CNE&W #15; PR&NE #15 - #1264 - #15; CNE (1st) #15 - #211)

HCW 16

Cost $12,500.00 new and was delivered to the H&CW RR on 11 Oct 1882. Was rebuilt with new BLW boiler in July 1900 at the Hartford, CT shops at a cost of $5231.25. Was condemned on 20 Nov 1919. (To CNE&W #16; PR&NE #16 - #1265 - #16; CNE (1st) #16 - #220 – (2nd) #35)

HCW 17

Was rebuilt with an extended front end July 1890 and reboilered in Apr 1901 at a cost of $5650.54. Was condemned in Oct 1925. (To CNE&W #17; PR&NE #17 - #1266 - #17; CNE (1st) #17 - #221 – (2nd) #36)

HCW 18

Cost new $9,572.00. Was reboilered in Jul 1901 and fitted with extended saddle at a cost of $5061.02. Was condemned in July 1926. (To CNE&W #18; PR&NE #18 - #1267 - #18; CNE (1st) #18 - #222 – (2nd) #37)

HCW 19

Cost new $9,825.00. Was condemned on 31 Dec 1911 for scrap value of $300.00. (To CNE&W (1st) #19 – (2nd) #22; PR&NE #22; CNE (1st) #22 - (2nd) #1)

Central New England & Western Railroad

23 July 1889 – 27 July 1892

Locomotive Roster Notes

| NBR | Name | Builder | Const # | Build Date | Delivery Date | Cylinder Size | Driver Diameter | Wheel Arrangement | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | State Line | Rogers L&M Co. | *1869 | 3/20/1871 | 9/15/1871 | 15" x 22" | 48 3/4" | 2-4-4T | CNEW 1 |
| 2 |  | Danforth L&M | 960 | 9/16/1873 |  | 16" x 24" | 46" | 0-4-2T | CNEW 2 |
| 3 | Winsted | Rogers L&M Co. | *1955 | 10/03/1871 | 10/12/1871 | 16" x 24" | 60" | 4-4-0 | CNEW 3 |
| 4 | Salisbury | Rogers L&M Co. | *1968 | 10/28/1871 | 11/6/1871 | 15" x 22" | 60" | 4-4-0 | CNEW 4 |
| 5 | Norfolk | Rogers L&M Co. | *1969 | 10/30/1871 | 11/1871 | 16" x 24" | 60" | 4-4-0 | CNEW 5 |
| 6 | Bloomfield | Rogers L&M Co. | *1838 |  | 1/9/1871 | 16" x 22" | 66" | 4-4-0 | CNEW 6 |
| 7 | Canton | Rogers L&M Co. | *2079 | 6/7/1872 |  | 17" x 22" | 54" | 4-4-0 | CNEW 7 |
| 8 | Canaan | Rogers L&M Co. | *2080 | 6/10/1872 |  | 17" x 22" | 54" | 4-4-0 | CNEW 8 |
| 9 | Simsbury | Rogers L&M Co. | 2363 | 11/17/1873 |  | 17" x 22" | 54" | 4-4-0 | CNEW 9 |
| 10 | Tariffville | Rogers L&M Co. | 2429 |  | 12/10/1871 | 17" x 22" | 60" | 4-4-0 | CNEW 10 |
| 1st 11 |  | Danforth L&M | 943 | 8/18/1873 |  | 17" x 24" | 60" | 4-4-0 | CNEW 11 |
| 2nd 11 |  | Baldwin L.W. | 12413 | 1/11/1892 |  | 21" x 24" | 62" | 4-6-0 | CNEW 12 |
| 1st 12 |  | Danforth L&M | 960 | 9/16/1873 |  | 16" x 24" | 46" | 0-4-2T | CNEW 13 |
| 2nd 12 |  | Baldwin L.W. | 12412 | 1/9/1892 |  | 21" x 24" | 62" | 4-6-0 | CNEW 14 |
| 1st 13 |  | Danforth L&M | 974 | 11/12/1873 |  | 17" x 24" | 60" | 4-4-0 | CNEW 15 |
| 2nd 13 |  | Baldwin L.W. | 12390 | 12/31/1891 |  | 21" x 24" | 62" | 4-6-0 | CNEW 16 |
| 14 |  | Baldwin L.W. | 12391 | 12/31/1891 |  | 21" x 24" | 62" | 4-6-0 | CNEW 17 |
| 15 |  | Rogers L&M Co. | 3052 | 8/7/1882 |  | 18" x 24" | 60" | 4-4-0 | CNEW 18 |
| 16 |  | Rogers L&M Co. | 3096 | 9/30/1882 |  | 18" x 24" | 62" | 4-4-0 | CNEW 19 |
| 17 |  | Rogers L&M Co. | 3299 | 7/6/1883 |  | 18" x 24" | 62" | 4-4-0 | CNEW 20 |
| 18 |  | Rogers L&M Co. | 3346 | 9/10/1883 |  | 18" x 24" | 62" | 4-4-0 | CNEW 21 |
| 1st 19 |  | Rogers L&M Co. | 4108 | 2/11/1889 |  | 19" x 24" | 56" | 2-6-0 | CNEW 22 |
| 2nd 19 |  | Baldwin L.W. | 9572 | 10/1888 |  | 18" x 24" | 63" | 4-4-0 | CNEW 23 |
| 20 |  | Baldwin L.W. | 11208 | 9/23/1890 |  | 18" x 24" | 63" | 4-4-0 | CNEW 24 |
| 21 |  | Baldwin L.W. | 9571 | 10/1888 |  | 18" x 24" | 63" | 4-4-0 | CNEW 25 |
| 1st 22 |  | Baldwin L.W. | 9572 | 10/1888 |  | 18" x 24" | 63" | 4-4-0 | CNEW 26 |
| 2nd 22 |  | Rogers L&M Co. | 4108 | 2/11/1889 |  | 19" x 24" | 56" | 2-6-0 | CNEW 27 |
| 23 |  | Baldwin L.W. | 9828 | 2/1889 |  | 19" x 24" | 54" | 2-6-0 | CNEW 28 |
| 24 |  | Baldwin L.W. | 9863 | 3/1889 |  | 20" x 24" | 50" | 2-8-0 | CNEW 29 |
| 25 |  | Baldwin L.W. | 9860 | 3/1889 |  | 20" x 24" | 50" | 2-8-0 | CNEW 30 |
| 26 |  | Baldwin L.W. | 9861 | 3/1889 |  | 20" x 24" | 50" | 2-8-0 | CNEW 31 |
| 27 |  | Baldwin L.W. | 11217 | 9/25/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 32 |
| 28 |  | Baldwin L.W. | 11214 | 9/24/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 33 |
| 29 |  | Baldwin L.W. | 10839 | 4/29/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 34 |
| 30 |  | Baldwin L.W. | 10843 | 4/30/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 35 |
| 31 |  | Baldwin L.W. | 11476 | 12/23/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 36 |
| 32 |  | Baldwin L.W. | 11477 | 12/23/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 37 |
| 33 |  | Baldwin L.W. | 11478 | 12/23/1890 |  | 20" x 24" | 50" | 2-8-0 | CNEW 38 |
| 41 |  | Danforth L&M | 943 | 8/18/1873 |  | 17" x 24" | 60" | 4-4-0 | CNEW 39 |
| 43 |  | Danforth L&M | 974 | 11/12/1873 |  | 17" x 24" | 60" | 4-4-0 | CNEW 40 |

(*) Indicates unofficial construction number as Rogers apparently used none between Jun 1856 and Nov 1872. (R&LHS Bulletin #167 – Fall, 1992)

CNEW 1

Had headlights at both ends. Used mainly as a switcher in Hartford, CT. Was shopped at Hartford in Sep 1890. Reportedly shipped by flatcar from storage in Canaan, CT to the P&R Railway’s Reading, PA shops in mid 1893 and was scrapped. (See CW Note 02) (Was CW #1; HC&W #1; To PR&NE (1st) #1)

CNEW 2

Used as a switcher in both Maybrook, NY and Hartford, CT. Renumbered from CNE&W RR (1st) #12 to #2 in Jan 1892 after receipt of new BLW 4-6-0 (2nd) #12. Was in a condemned state at Hartford in Nov 1894 but was repaired and returned to service. Was again in a condemned state in Hartford on the CNE Railway takeover of the PR&NE in Jan 1899 and was scrapped soon after. (See R&C RR note 4) (was R&C (1st) #3 – (2nd) #2; H&CW #12; To PR&NE #2 - #1251 - #2)

CNEW 3

Was shopped at Hartford in Oct 1890. Was taken to the Reading, PA shops in fall of 1893 for major repairs that were never made. Written off by the PR&NE RR in 4th quarter of 1895 and presumably scrapped by the P&R Railway soon after. (See CW note 4) (Was CW #3; H&CW #3; To PR&NE #3)

CNEW 4

Was removed from road service in late 1892 and used as a stationary boiler in Hartford, CT to heat passenger cars. The #4 spot replaced by a new BLW 4-4-0 (2nd) #4 bought by the PR&NE RR in Dec 1893. Was condemned and sold for scrap to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. and shipped to them on 18 Mar 1899 for scrapping. (See CW RR note 5) (Was CW #4; H&CW #4; To PR&NE (1st) #4)

CNEW 5

Was wrecked at Modena, NY on 13 Nov 1892. Was out of service on the CNE Railway takeover of the PR&NE in Jan 1899 but was repaired and returned to service. Was used on Poughkeepsie Bridge trolley shuttle service in Mar 1899 while regular loco was being repaired. Was sold to the American Iron & Steel Mfg. Co. and sent to Lebanon, PA for scrapping on 16 Jan 1901. (See CW RR Note 6) (Was CW #5; H&CW #5; To PR&NE #5 - #1254 - #5; CNE (1st) #5)

CNEW 6

Was condemned on 9 Mar 1899 and sold to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. for scrapping. It was shipped there on 18 Mar 1899. (See CW RR Note 7) (Was CW #6; H&CW #6; To PR&NE #6 - #1255 - #6; CNE (1st) #6)

CNEW 7

Was condemned and sent to the P&R Railway’s Reading, PA shops in late 1893 for scrapping. The #7 spot was replaced by new BLW 4-4-0 (2nd) #7 built for the PR&NE in Dec 1893. (See CW RR Note 8) (Was CW #7; H&CW #7; To PR&NE (1st) #7)

CNEW 8

Was condemned and sent to the P&R Railway’s Reading, PA shops in late 1893 for scrapping. The #8 spot was replaced by new BLW 4-4-0 (2nd) #8 built for the PR&NE in Dec 1893. (See CW RR Note 9) (Was CW #8; H&CW #8; To PR&NE (1st) #8)

CNEW 9

Was taken to the P&R Railway’s Reading, PA shops in the fall of 1893 for repairs that were never made.. Was written off by the PR&NE in the 4th quarter of 1895 and presumably scrapped by the P&R Railway soon after. (See CW RR Note 10) (Was CW #9; H&CW #9; To PR&NE #9)

CNEW 10

Was rebuilt at the CNE&W’s Hartford, CT shops in Oct 1890. Reportedly was out of service for four years from late 1893 until late 1897. Was rebuilt again at the Hartford, CT shops in Jun 1898 and in Feb 1902. Was sold on 2 Nov 1906 to an I.U. Harris (scrap dealer) of Hartford. CT for $625.00. (See CW RR Note 10) (Was CW #10; H&CW #10; To PR&NE #10; CNE (1st) #10 - #210)

CNEW 11

Was renumbered from (1st) #11 to #41 by the CNE&W RR on delivery of the new BLW 4-6-0 (2nd) #11 in Jan 1892. Was scrapped by the P&R Railway soon after it’s takeover of the CNE&W RR in Aug 1892. (See R&C note 3) (was R&C (1st) #2 – (2nd) #1; HC&W #11)

CNEW 12

BLW Class 10/36D-68. Cost new $10,285.00 including a train signal system. Was wrecked in the Canaan, CT yards on 27 July 1892 and again in East Canaan, CT on 29 Jun 1893. Was condemned in July 1925. (To PR&NE #11 - #1260 - #11; CNE (1st) #11 – (2nd) #25 - #47)

CNEW 13

Was renumbered from CNE&W RR (1st) #12 to vacant (2nd) #2 spot in Jan 1892 on delivery of new BLW 4-6-0 (2nd) #12. Used as a switcher in both Maybrook and Hartford. Was in a condemned state at Hartford in Nov 1894 but was shopped and returned to service. Was again in a condemned state upon the CNE Railway’s takeover of the PR&NE RR in Jan 1899 and presumed scrapped soon after. (See R&C note 4) (Was R&C (1st) #3 – (2nd) #2; H&CW #12; To PR&NE #2 - #1251 - #2)

CNEW 14

BLW Class 10/36D-67. Cost new $10,285.00 including a train signal system. Was condemned as of 31 Dec 1915 for $300.00 scrap value. (To PR&NE #12 - #1261 - #12; CNE (1st) #12 – (2nd) #26)

CNEW 15

Was renumbered from CNE&W RR (1st) #13 to #43 on delivery of new BLW 4-6-0 (2nd) #13 in Jan 1892. Scrapped by the P&R Railway soon after it’s takeover of the CNE&W RR in Aug 1892. (See R&C note 5) (was R&C (1st) #4 – (2nd) #3; HC&W #13)

CNEW 16

BLW Class 10/36D-66. Cost new $10,250.00. Received major overhaul at BLW in Philadelphia, PA in Dec 1893 and Jan 1894 costing $2,425.70. Was condemned 30 Apr 1915 for scrap value of $300.00. (To PR&NE #13 - #1262 - #13; CNE (1st) #13 – (2nd) #27)

CNEW 17

BLW Class 10/36D-65. Cost new $10,250.00. Was wrecked at Copake, NY on 28 Mar 1893. Condemned in Apr 1926. (To PR&NE #14 - #1263 - #14; CNE (1st) #14 – (2nd) #28 - #48)

CNEW 18

Was used primarily on the Rhinecliff Branch due to it’s light weight. Rebuilt by the Rogers Locomotive Works in the spring of 1892. Was condemned in Apr 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $200.00. (See H&CW note 15) (Was H&CW #15; To PR&NE #15 - #1264 - #15; CNE (1st) #15 - #211)

CNEW 19

Rebuilt with new Baldwin boiler in July 1900 at the CNE Railway’s Hartford, CT shops at a cost of $5231.25. Was condemned on 20 Nov 1919. (See H&CW note 16) (Was H&CW #16; To PR&NE #16 - #1265 - #16; CNE (1st) #16 - #220 – (2nd) #35)

CNEW 20

Was rebuilt with an extended front end July 1890. Reboilered at the CNE Railway’s Hartford, CT shops in Apr 1901 at a cost of $5650.54. Was condemned in Oct 1925. (See H&CW note 17) (Was H&CW #17; To PR&NE #17 - #1266 - #17; CNE (1st) #17 - #221 – (2nd) #36)

CNEW 21

Was reboilered at the CNE Railway’s Hartford, CT shops in Jul 1901 and fitted with extended saddle at a cost of $5061.02. Was condemned in July 1926. (See H&CW note 18) (Was H&CW #18; To PR&NE #18 - #1267 - #18; CNE (1st) #18 - #222 – (2nd) #37)

CNEW 22
Was renumbered in late 1890 or early 1891 by the CNE&W RR from (1st) #19 to (2nd) #22 in order to fall in line in proper order with other 2-6-0- #23. Was condemned on 31 Dec 1911 for value of $300.00 and scrapped soon after. (See H&CW note 19) (Was H&CW #19; To PR&NE #22; CNE (1st) #22 – (2nd) #1)

CNEW 23

Was renumbered in late 1890 or early 1891 by the CNE&W RR from (1st) #22 to (2nd) #19 in order to fall in line in proper order with other 4-4-0’s #18 and #20. Was condemned by the CNE Railway on 31 Dec 1915 for value of $300.00 and scrapped soon after. (See HC RR note 2) (Was HC #2; To PR&NE #19 - #1268 - #19; CNE (1st) #19 - #223 - (2nd) #31)

CNEW 24

BLW Class 8/30C-344. Purchased new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Shipped new from BLW in Philadelphia, PA on 30 Sep 1890 for delivery to the CNE&W RR at Campbell Hall, NY. Reported to have been a poor steamer and sent to the P&R Railway’s Reading, PA shops in Dec 1892 for rebuilding. Was condemned by the CNE Railway on 30 Apr 1915 for $250.00 scrap value. (To PR&NE #20 - #1269 - #20; CNE (1st) #20 - #224 – (2nd) #32)

CNEW 25

Was wrecked at Salisbury, CT on 24 Oct 1890 and again at Clintondale, NY on 23 Oct 1892. Taken to the P&R Railway’s Reading, PA shops in Nov 1892 for rebuilding. Was condemned by the CNE Railway on 30 Sep 1919 for $400.00 scrap value. (See HC RR note 1) (Was HC #1; To PR&NE #21 - #1270 - #21; CNE (1st) #21 - #225 - (2nd) #33)

CNEW 26

See above CNEW 23. Was renumbered in late 1890 or early 1891 by the CNE&W RR from (1st) #22 to (2nd) #19 in order to fall in line in proper order with other 4-4-0’s #18 and #20. Was condemned by the CNE Railway on 31 Dec 1915 for value of $300.00 and scrapped soon after. (See HC RR note 2) (Was HC #2; To PR&NE #19 - #1268 - #19; CNE (1st) #19 - #223 - (2nd) #31)

CNEW 27

See above CNEW 22. Was renumbered in late 1890 or early 1891 by the CNE&W RR from (1st) #19 to (2nd) #22 in order to fall in line in proper order with other 2-6-0- #23. Was condemned on 31 Dec 1911 for value of $300.00 and scrapped soon after. (See H&CW note 19) (Was H&CW #19; To PR&NE #22; CNE (1st) #22 – (2nd) #1)

CNEW 28

Was wrecked at Salisbury, CT on 24 Oct 1890. Condemned by the CNE Railway on 1 Jul 1914 for $350.00 scrap value. (See HC RR note 3) (Was HC #3; To PR&NE #23 - #1272 - #23; CNE (1st) #23 – (1st) #2 – (1st) #104 – (1st) 106 - #77)

CNEW 29

Sold to Fitzhugh-Luther & Co. (locomotive broker) by the CNE Railway on 22 Apr 1905 for $1850.00 who re-sold it to the Duluth, Rainy Lake & Winnipeg RR. Line became the Duluth, Winnipeg & Pacific RR in 1912. Final disposition unknown. (See HC RR note 4) (Was HC #4; To PR&NE #24 - #1273 - #24; CNE (1st) #24)

CNEW 30

Was wrecked at Lakeville, CT on 26 Jun 1893. Condemned by the CNE Railway on 30 Apr 1915 for $350.00 scrap value. (See HC RR note 5) (Was HC #5; To PR&NE #25 - #1274 - #25; CNE (1st) #25; NH #257; again CNE (1st) #25 - #75)

CNEW 31

Was shipped to BLW in Philadelphia, PA to repair wreck damage sustained at New Hartford, CT in Dec 1894. Sold by the CNE Railway to the E.H. Wilson Co. (locomotive broker) in Sep 1907 for $2000.00. Final disposition unknown. (See HC RR note 6) (Was HC #6; To PR&NE #26 - #1275 - #26; CNE (1st) #26; NH #258; again CNE (1st) #26)

CNEW 32

BLW Class 10/34E-1228. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 2 Oct 1890 to Hartford, CT. Cost new $9,695.00. Sold on 14 Sep 1906 to Fitzhugh-Luther & Co. (locomotive broker) for $1750.00 and by them, in turn, to the Spokane & Inland Empire RR (#6). Resold to the Pacific & Eastern RR (2nd #5) of Medford, OR in 1911. This line became the Brownlee Olds Lbr. Co. in 1921. Final disposition unknown. (To PR&NE #27 - #1276 - #27; CNE (1st) #27)

CNEW 33

BLW Class 10/34E-1228. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 2 Oct 1890 to Hartford, CT. Cost new $9,695.00. Was wrecked at Clintondale, NY on 23 Oct 1892 and again at Stissing Jct, NY on 11 May 1893. Was sold by the CNE Railway in May 1903 to Fitzhugh & Co. (locomotive broker) for $2800.00 and by them to Weyerhauser Lumber Co. to become Hawthorne, Nebagamon & Superior RR (Wisc.) #2. Was sold in 1908 to the Mesabe Southern RR (#4) in Minn. Sold again in 1912 to the Duluth & Northeastern RR (their #12) in Minn. Was wrecked and scrapped in 1924 but it’s boiler was used in the D&NE’s roundhouse at Cloquet, Minn. For sometime after. (To PR&NE #28 - #1277 - #28; CNE (1st) #28)

CNEW 34

BLW Class 10/34E-1060. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 5 May 1890 via the P&R Railway to Campbell Hall, NY. Cost new $9,615.00. Was sold in Nov 1902 by the CNE Railway to The Males Co. (locomotive broker) for $2300.00 and resold by them to the New York &Pennsylvania RR in May 1903 where it became their #29. Remained on the New York &Pennsylvania RR roster until at least 1929 and probably scrapped. (To PR&NE #29 - #1278 - #29; CNE (1st) #29)

CNEW 35

BLW Class 10/34E-1061. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 5 May 1890 via the P&R Railway to Campbell Hall, NY. Cost new $9,615.00. Was sold in Nov 1902 by the CNE Railway to The Males Co. (locomotive broker) for $2300.00 and resold by them to the Detroit Southern RR in May or June 1903 for use on their Ohio Southern Division where it became #29. In May 1905 these lines became the Detroit, Toledo & Ironton RR. Was scrapped about 1921. (To PR&NE #30 - #1279 - #30; CNE (1st) #30)

CNEW 36

BLW Class 10/34E-1240. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 31 Dec 1890 to Hartford, CT. Cost new $9,695.00. Was on the freight carrying a car of dynamite that blew up in Barkhampsted, CT 20 Sep 1892. Was condemned by the CNE Railway on 20 Oct 1913 for $323.00 scrap value. (To PR&NE #31; CNE (1st) #31)

CNEW 37

BLW Class 10/34E-1241. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 31 Dec 1890 to Hartford, CT. Cost new $9,695.00. Was condemned by the CNE Railway on 31 Dec 1912 for $500.00 scrap value. (To PR&NE #32 - #1281 - #32; CNE (1st) #32)

CNEW 38

BLW Class 10/34E-1242. Bought new by the Delaware & New England RR (a paper organization) but lettered for the CNE&W RR. Was shipped new from BLW in Philadelphia, PA on 31 Dec 1890 to Hartford, CT. Cost new $9,695.00. Was wrecked at Simsbury , CT on 28 Mar 1893 and shipped to BLW in Philadelphia, PA for major repairs in Dec 1894. Was sold by the CNE Railway in May 1903 to Fitzhugh & Co. (locomotive broker) for $2800.00 and resold by them to the Weed Lumber Co., Weed, CA in Oct 1903 where it became their #3. Sold to the California Northeastern RR (locally called the “CNE”) in Jul 1905 becoming their #3 as well. The California Northeastern RR was eventually taken over by the Southern Pacific RR and this locomotive became S.P. (Class C-6) (2nd) #2500 on 1 Sep 1910. Was condemned on 31 Jul 1927 and scrapped in Sacremento, CA on 23 Sep 1927. (To PR&NE #33 - #1282 - #33; CNE (1st) #33)

CNEW 39

Renumbered from CNE&W RR (1st) #11 to #41 in Jan 1892 upon delivery of new BLW 4-6-0 (2nd) #11. Was scrapped by the P&R Railway shortly after their takeover of the CNE&W RR in Aug 1892. (See R&C RR note 3) (Was R&C #2 – (2nd) #1; H&CW #11)

CNEW 40

Renumbered from CNE&W RR (1st) #13 to #43 in Jan 1892 upon delivery of new BLW 4-6-0 (2nd) #13. Was scrapped by the P&R Railway shortly after their takeover of the CNE&W RR in Aug 1892. (See R&C RR note 5) (Was R&C #4 – (2nd) #3; H&CW #13)

Philadelphia, Reading & New England Railroad

7/28/1892 to 1/11/1899

Locomotive Roster Notes

| Aug 1892 Number | Jan 1893 Number | Feb 1894 Number | Name | Builder | Const # | Build Date | Delivery Date | Cylinder Size | Driver Diameter | Wheel Arrangement | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1st) 1 | %1250 |  | State Line | Rogers L&M Co. | *1869 | 3/20/1871 | 9/15/1871 | 15" x 22" | 48 3/4" | 2-4-4T | PRNE 1 |
|  |  | (2nd) 1 |  | H.K. Porter | 1075,76,or78 |  | 8or9/1889 | 10" x 14" | 22" | 04-2T | PRNE 2 |
| 2 | +1251 | 2 |  | Danforth L&M | 960 | 9/16/1873 |  | 16" x 24" | 46" | 0-4-2T | PRNE 3 |
| 3 | %1252 | 3 | Winsted | Rogers L&M Co. | *1955 | 10/03/1871 | 10/12/1871 | 16" x 24" | 60" | 4-4-0 | PRNE 4 |
| (1st) 4 | %1253 |  | Salisbury | Rogers L&M Co. | *1968 | 10/28/1871 | 11/6/1871 | 15" x 22" | 60" | 4-4-0 | PRNE 5 |
|  |  | (2nd) 4 |  | Baldwin L.W. | 13857 |  | 12/1893 | 12+20" x 24" | 68" | 4-4-0 | PRNE 6 |
| 5 | +1254 | 5 | Norfolk | Rogers L&M Co. | *1969 | 10/30/1871 | 11/1871 | 16" x 24" | 60" | 4-4-0 | PRNE 7 |
| 6 | +1255 | 6 | Bloomfield | Rogers L&M Co. | *1838 |  | 1/9/1871 | 16" x 22" | 66" | 4-4-0 | PRNE 8 |
| (1st) 7 | %1256 |  | Canton | Rogers L&M Co. | *2079 | 6/7/1872 |  | 17" x 22" | 54" | 4-4-0 | PRNE 9 |
|  |  | (2nd) 7 |  | Baldwin L.W. | 13858 | 12/1893 |  | 12+20"x24" | 68" | 4-4-0 | PRNE 10 |
| (1st) 8 | %1257 |  | Canaan | Rogers L&M Co. | *2080 | 6/10/1872 |  | 17" x 22" | 54" | 4-4-0 | PRNE 11 |
|  |  | (2nd) 8 |  | Baldwin L.W. | 13859 | 12/1893 |  | 12+20"x24" | 68" | 4-4-0 | PRNE 12 |
| 9 | %1258 | 9 | Simsbury | Rogers L&M Co. | 2363 | 11/17/1873 |  | 17" x 22" | 54" | 4-4-0 | PRNE 13 |
| 10 | (1259) | 10 | Tariffville | Rogers L&M Co. | 2429 |  | 12/10/1871 | 17" x 22" | 60" | 4-4-0 | PRNE 14 |
| 11 | +1260 | 11 |  | Baldwin L.W. | 12413 | 1/11/1892 |  | 21" x 24" | 62" | 4-6-0 | PRNE 15 |
| 12 | +1261 | 12 |  | Baldwin L.W. | 12412 | 1/9/1892 |  | 21" x 24" | 62" | 4-6-0 | PRNE 16 |
| 13 | +1262 | 13 |  | Baldwin L.W. | 12390 | 12/31/1891 |  | 21" x 24" | 62" | 4-6-0 | PRNE 17 |
| 14 | +1263 | 14 |  | Baldwin L.W. | 12391 | 12/31/1891 |  | 21" x 24" | 62" | 4-6-0 | PRNE 18 |
| 15 | +1264 | 15 |  | Rogers L&M Co. | 3052 | 8/7/1882 |  | 18" x 24" | 60" | 4-4-0 | PRNE 19 |
| 16 | +1265 | 16 |  | Rogers L&M Co. | 3096 | 9/30/1882 |  | 18" x 24" | 62" | 4-4-0 | PRNE 20 |
| 17 | +1266 | 17 |  | Rogers L&M Co. | 3299 | 7/6/1883 |  | 18" x 24" | 62" | 4-4-0 | PRNE 21 |
| 18 | +1267 | 18 |  | Rogers L&M Co. | 3346 | 9/10/1883 |  | 18" x 24" | 62" | 4-4-0 | PRNE 22 |
| 19 | +1268 | 19 |  | Baldwin L.W. | 9572 | 10/1888 |  | 18" x 24" | 63" | 4-4-0 | PRNE 23 |
| 20 | +1269 | 20 |  | Baldwin L.W. | 11208 | 9/23/1890 |  | 18" x 24" | 63" | 4-4-0 | PRNE 24 |
| 21 | +1270 | 21 |  | Baldwin L.W. | 9571 | 10/1888 |  | 18" x 24" | 63" | 4-4-0 | PRNE 25 |
| 22 | (1271) | 22 |  | Rogers L&M Co. | 4108 | 2/11/1889 |  | 19" x 24" | 56" | 2-6-0 | PRNE 26 |
| 23 | +1272 | 23 |  | Baldwin L.W. | 9828 | 2/1889 |  | 19" x 24" | 54" | 2-6-0 | PRNE 27 |
| 24 | +1273 | 24 |  | Baldwin L.W. | 9863 | 3/1889 |  | 20" x 24" | 50" | 2-8-0 | PRNE 28 |
| 25 | +1274 | 25 |  | Baldwin L.W. | 9860 | 3/1889 |  | 20" x 24" | 50" | 2-8-0 | PRNE 29 |
| 26 | +1275 | 26 |  | Baldwin L.W. | 9861 | 3/1889 |  | 20" x 24" | 50" | 2-8-0 | PRNE 30 |
| 27 | +1276 | 27 |  | Baldwin L.W. | 11217 | 9/25/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 31 |
| 28 | +1277 | 28 |  | Baldwin L.W. | 11214 | 9/24/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 32 |
| 29 | +1278 | 29 |  | Baldwin L.W. | 10839 | 4/29/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 33 |
| 30 | +1279 | 30 |  | Baldwin L.W. | 10843 | 4/30/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 34 |
| 31 | (1280) | 31 |  | Baldwin L.W. | 11476 | 12/23/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 35 |
| 32 | +1281 | 32 |  | Baldwin L.W. | 11477 | 12/23/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 36 |
| 33 | +1282 | 33 |  | Baldwin L.W. | 11478 | 12/23/1890 |  | 20" x 24" | 50" | 2-8-0 | PRNE 37 |
|  |  | 34 |  | Baldwin L.W. | 13847 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 38 |
|  |  | 35 |  | Baldwin L.W. | 13848 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 39 |
|  |  | 36 |  | Baldwin L.W. | 13849 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 40 |
|  |  | 37 |  | Baldwin L.W. | 13850 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 41 |
|  |  | 38 |  | Baldwin L.W. | 13851 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 42 |
|  |  | 39 |  | Baldwin L.W. | 13852 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 43 |
|  |  | 40 |  | Baldwin L.W. | 13853 | 11/1893 |  | 13.5"+23"x24" | 51" | 2-8-0 | PRNE 44 |
| 41 |  |  |  | Danforth L&M | 943 | 8/18/1873 |  | 17" x 24" | 60" | 4-4-0 | PRNE 45 |
| 43 |  |  |  | Danforth L&M | 974 | 11/12/1873 |  | 17" x 24" | 60" | 4-4-0 | PRNE 46 |

The FIRST Column numbering indicates existing locomotives with assigned numbers on hand upon the P&R Railway’s takeover of the CNE&W RR on 1 Aug 1892. The SECOND Column indicates the renumbering of locomotives as ordered by the P&R Railway’s headquarters in Philadelphia, PA beginning in early 1893 in order to eliminate the possibility of duplicate numbering within the P&R system. The THIRD Column indicates the renumbering of locomotives (with exceptions noted below) beginning in Feb 1894 back to their original numbers after the P&R lost control of the PR&NE RR in late 1893.

The “+” sign before and after a locomotive number indicates that written and/or photo evidence exists as to the actual renumbering of that particular locomotive in the “1200” series. The “( )” bracketing a locomotive number in the middle column indicates that this particular locomotive, although a new number had been assigned to it, did not get renumbered in to the “1200” series as it had not as yet gone through the shops prior to the order to revert back to the original numbering system. The “%” mark before and after a locomotive number indicates the P&R ordered number assigned to this particular locomotive; however, it was never applied due to it’s having been removed from service or scrapped before going through the shops.

(*) Indicates unofficial construction number as Rogers apparently used none between Jun 1856 and Nov 1872 . (R&LHS Bulletin #167 – Fall, 1992)

(@) Indicates that the decision was made to revert back to single and double digit locomotive numbering in Oct 1893 and then the ten new BLW compounds (three 4-4-0’s and seven 2-8-0’s) delivered in Dec 1893 were numbered 4,7,8,34,35,36,37,38,39 & 40 at the factory. Most other existing locomotives were not renumbered until new plates prepared by BLW and Rogers were delivered to the Hartford, CT shops in Feb 1894.

PRNE 1

Said to have been out of service on P&R Railway’s takeover of the CNE&W RR and was taken out of storage in Canaan, CT and placed on a flatcar and brought to the P&R’s Reading, PA shops in mid 1893 where it was scrapped. (see CW note 2) (Was CW #1; H&CW #1; CNE&W #1

PRNE 2

Was one of three locomotives originally built for the North Hudson County Railway (#3, #4, or #6) of Hoboken, NJ and was bought used in July 1897 from the New Jersey Car & Equipment Co. Was a “dummy” (enclosed) locomotive used in hauling trolleys from Pratts Mills (west of Highland, NY) over the Poughkeepsie Bridge in to the city of Poughkeepsie. Taken to Hartford, CT in early 1902 and used in the construction of the Springfield Branch. Was sold in May 1903 to F.M. Hicks Co. (locomotive broker) of Chicago, IL for $800.00. Final disposition unknown. (To CNE (1st) #1)

PRNE 3

Used as a yard switcher in Hartford, CT but was condemned in Nov 1894. Later rebuilt and returned to service in Hartford, CT. Was again in a condemned state upon the CNE Railway’s takeover of the PR&NE in Jan 1899 and undoubtedly was scrapped shortly afterward. (See R&C note 4) (Was R&C (1st) #3 – (2nd) #2; H&CW #12; CNE&W (1st) #12 - #2)

PRNE 4

Was taken to the Reading, PA shops in fall of 1893 for major repairs that were never made. Written off by the PR&NE RR in 4th quarter of 1895 and presumably scrapped by the P&R Railway around that time. (PR&NE Class “I”) (See CW note 4) (Was CW #3; H&CW #3; CNE&W #3)

PRNE 5

Was removed from road service in late 1892 and used as a stationary boiler to heat passenger cars in Hartford, CT. Was sold for scrap by the CNE Railway to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. and shipped there on 18 Mar 1899. (PR&NE Class “I”) (See CW note 5) (Was CW #4; H&CW #4; CNE&W #4)

PRNE 6

BLW Class 8 18/34C-10. Was shipped new from BLW in Philadelphia, PA on Thursday, 14 Dec 1893 to Hartford, CT. Cylinders were simpled to 18”x24” and drivers extended to 69” during rebuilding by the CNE Railway at the Hartford, CT shops in 1908. Was condemned in Nov 1926. (To CNE (1st) #4 - #200 - #229 – (2nd) #41)

PRNE 7

Was wrecked at Modena, NY on 13 Nov 1892. Was used on Poughkeepsie Bridge trolley shuttle service in Mar 1899 while regular loco was being repaired. Was out of service on the CNE Railway takeover of the PR&NE in Jan 1899 but was repaired and returned to service. Was sold to the American Iron & Steel Mfg. Co. and sent to Lebanon, PA for scrapping on 16 Jan 1901. (PR&NE Class “I”) (See CW note 6) (Was CW #5; H&CW #5; CNE&W #5; To CNE (1st) #5)

PRNE 8

Was condemned on 9 Mar 1899 and sold to the Pennsylvania Bolt & Nut Co. in Lebanon, Pa. where it was sent for scrap on 18 Mar 1899. (See CW note 7) (Was CW #6; H&CW #6; CNE&W #6; To CNE (1st) #6)

PRNE 9

Was condemned in mid-1893 and sent to the P&R Railway’s Reading, PA shops in Sep 1893 for scrapping. (See CW note 8) (Was CW #7; H&CW #7; CNE&W #7)

PRNE 10

BLW Class 8 18/34C-11. Was shipped new from BLW in Philadelphia, PA on Monday, 18 Dec 1893 to Hartford, CT. Cylinders were simpled to 18”x24” and drivers extended to 69” during rebuilding by the CNE Railway at the Hartford, CT shops in 1911. Was condemned on 10 Oct 1926. (To CNE (1st) #7 - #201 - #231 – (2nd) #43)

PRNE 11

Was condemned in first half of 1893 and sent to the P&R Railway’s Reading, PA shops for scrapping. (See CW note 9) (Was CW #8; H&CW #8; CNE&W #8)

PRNE 12

BLW Class 8 18/23C-12. Was shipped new from BLW in Philadelphia, PA on Monday, 18 Dec 1893 to Hartford, CT. Cylinders were simpled to 18”x24” and drivers extended to 69” during rebuilding by the CNE Railway at the Hartford, CT shops in 1909. Was condemned in Nov 1925. (To CNE #8 - #202 - #230 – (2nd) #42)

PRNE 13

Was taken to the Reading, PA shops in fall of 1893 for major repairs that were never made. Written off by the PR&NE RR in 4th quarter of 1895 and presumably scrapped by the P&R Railway around that time. (PR&NE Class “J”) (See CW note 10) (Was CW #9; H&CW #9; CNE&W #9)

PRNE 14

Was reportedly was out of service for four years from late 1893 until late 1897. Rebuilt at the Hartford, CT shops in Jun 1898 and in Feb 1902. Was condemned by the CNE Railway and sold to an I.U. Harris (scrap dealer) of Hartford. CT on 2 Nov 1906 for $625.00. (See CW note 11) (Was CW #10; H&CW #10; CNE&W #10; To CNE (1st) #10 - #210)

PRNE 15

Was wrecked in the Canaan, CT yards on 27 July 1892 and again in East Canaan, CT on 29 Jun 1893. Was condemned by the CNE Railway in July 1925. (See CNEW note 12) (PR&NE Class “D”) (Was CNE&W (2nd) #11; To CNE (1st) #11 – (2nd) #25 - (2nd) #47)

PRNE 16

Was condemned by the CNE Railway on 31 Dec 1915 for $300.00 scrap value. (See CNEW note 14) (PR&NE Class “D”) (Was CNE&W (2nd) #12; To CNE (1st) #12 – (2nd) #26)

PRNE 17

Received major overhaul at BLW in Philadelphia, PA in Dec 1893 and Jan 1894 costing $2,425.70. Was condemned 30 Apr 1915 for scrap value of $350.00. (See CNEW note 16) (PR&NE Class “D”) (Was CNE&W (2nd) #13; To CNE (1st) #13 – (2nd) #27)

PRNE 18

Was wrecked at Copake, NY on 28 Mar 1893. Condemned by the CNE Railway in Apr 1926. (See CNEW note 17) (PR&NE Class “D”) (Was CNE&W #14; To CNE (1st) #14 – (2nd) #28 – (2nd) #48)

PRNE 19

Used primarily on the Rhinecliff Branch account of it’s light weight. Rebuilt in spring 1892 by Rogers Locomotive Works of Paterson, NJ. Was condemned by the CNE Railway in Apr 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $200.00. (See H&CW note 15) (PR&NE Class “E”) (Was H&CW #15; CNE&W #15; To CNE (1st) #15 – #211)

PRNE 20

Rebuilt with new BLW boiler by the CNE Railway at the Hartford, CT shops in July 1900 at a cost of $5231.25. Was condemned on 20 Nov 1919. (See H&CW note 16) (PR&NE Class “E”) (Was H&CW #16; CNE&W #16; To CNE (1st) #16 – #220 – (2nd) #35)

PRNE 21

Was rebuilt at the CNE Railway’s Hartford, CT shops with a new boiler in Apr 1901 at a cost of $5650.54. Was condemned in Oct 1925. (See H&CW note 17) (PR&NE Class “E”) (Was H&CW #17; CNE&W #17; To CNE (1st) #17 – #221 – (2nd) #36)

PRNE 22

Was rebuilt at the CNE Railway’s Hartford, CT shops with a new boiler (fitted with extended saddle) in Jul 1901 at a cost of $5061.02. Was condemned in July 1926. (See H&CW note 18) (PR&NE Class “E”) (Was H&CW #18; CNE&W #18; To CNE (1st) #18 – #222 – (2nd) #37)

PRNE 23

Was condemned by the CNE Railway on 31 Dec 1915 for $300.00 scrap value. (See HC RR note 2) (PR&NE Class “F”) (Was HC #2; CNE&W (1st) #22 – (2nd) #19; To CNE (1st) #19 - #223 – (2nd) #31)

PRNE 24

Reported to have been a poor steamer and sent to the P&R Railway’s Reading, PA shops in Dec 1892 for rebuilding. Was condemned by the CNE Railway on 30 Apr 1915 for $250.00 scrap value. (See CNEW note 24) (PR&NE Class “F”) (Was CNE&W #20; To CNE (1st) #20 – #224 – (2nd) #32)

PRNE 25

Was wrecked at Clintondale, NY on 23 Oct 1892 and taken to the P&R Railway’s Reading, PA shops in Nov 1892 for rebuilding. Was condemned by the CNE Railway on 30 Sep 1919 for $400.00 scrap value. (See HC RR note 01) (PR&NE Class “F”) (Was HC #1; CNE&W #21; To CNE (1st) #21 - #225 – (2nd) #33)

PRNE 26

Was condemned by the CNE Railway on 31 Dec 1911 for $300.00 scrap value. (See H&CW note 19) (PR&NE Class “G”) (Was H&CW #19; CNE&W (1st) #19 – (2nd) #22; To CNE (1st) #22 – (2nd) #1)

PRNE 27

Was condemned by the CNE Railway on 31 Dec 1911 for $350.00 scrap value. (See HC RR note 03) (PR&NE Class “H”) (Was HC #3; CNE&W #23; To CNE (1st) #23 – (1st) #2 – (1st) #104 – (1st) #106 – #77)

PRNE 28

Sold by the CNE Railway to Fitzhugh-Luther & Co. (locomotive broker) on 22 Apr 1905 for $1850.00 who re-sold it to the Duluth, Rainy Lake & Winnipeg RR. Line became the Duluth, Winnipeg & Pacific RR in 1912. Final disposition unknown. (See HC RR note 4) (PR&NE Class “B”) (Was HC #4; CNE&W #24; To CNE (1st) #24)

PRNE 29

Was wrecked at Lakeville, CT on 26 Jun 1893. Condemned by the CNE Railway on 30 Apr 1915 for $350.00 scrap value. (See HC RR note 5) (PR&NE Class “B”) (Was HC #5; CNE&W #25; To CNE (1st) #25; NH #257; again CNE (1st) #25 - #75)

PRNE 30

Wrecked in New Hartford, CT and was shipped to BLW in Philadelphia, PA in Dec 1894. Sold by the CNE Railway to the E.H. Wilson Co. (locomotive broker) in Sep 1907 for $2000.00. Final disposition unknown. (See HC RR note 6) (PR&NE Class “B”) (Was HC #6; CNE&W #26; To CNE (1st) #26; NH #258; again CNE (1st) #26)

PRNE 31

Was sold by the CNE Railway to Fitzhugh-Luther & Co. (locomotive broker) on 14 Sep 1906 for $1750.00. Resold by them to the Spokane & Inland Empire RR becoming their #6. Resold to the Pacific & Eastern RR of Medford, OR in 1911 becoming their (2nd) #5. This line became the Brownlee Olds Lumber. Co. in 1921. Final disposition unknown. (See CNEW note 32) (PR&NE Class “A”) (To CNE (1st) #27)

PRNE 32

Was wrecked at Clintondale, NY on 23 Oct 1892 and again at Stissing Jct, NY on 11 May 1893. Was sold by the CNE Railway in May 1903 to Fitzhugh & Co. (locomotive broker) for $2800.00 and by them to Weyerhauser Lumber Co. who used it on their Hawthorne, Nebagamon & Superior RR (Wisc.) as #2. Was sold in 1908 to the Mesabe Southern RR (their #4) in Minnesota. Was resold in 1912 to the Duluth & Northeastern RR (also in Minnesota) becoming their #12. Was wrecked and scrapped in 1924 but it’s boiler was used for sometime after in the D&NE’s roundhouse at Cloquet, Minnesota. (See CNEW note 33) (PR&NE Class “A”) (To CNE (1st) #28)

PRNE 33

Was sold in Nov 1902 by the CNE Railway to The Males Co. (locomotive broker) of New York City for $2300.00 and resold by them to the New York &Pennsylvania RR in May 1903 where it became their #29 as well. Remained on the New York &Pennsylvania RR roster until at least 1929 and probably was scrapped soon after. (See CNEW note 34) (PR&NE Class “A”) (To CNE (1st) #28)  (To CNE (1st) #29)

PRNE 34

Was sold in Nov 1902 by the CNE Railway to The Males Co. (locomotive broker) of New York City for $2300.00 and resold by them to the Detroit Southern RR in May or June 1903 for use on their Ohio Southern Division as #29. In May 1905 these lines became the Detroit, Toledo & Ironton RR. The locomotive was apparently scrapped about 1921. (See CNEW note 35) (PR&NE Class “A”) (To CNE (1st) #30)

PRNE 35

Was the locomotive on the freight carrying a car of dynamite that blew up in Barkhamsted, CT 20 Sep 1892. Was condemned by the CNE Railway on 20 Oct 1913 for $323.00 scrap value. (See CNEW note 36) (PR&NE Class “A”) (To CNE (1st) #31)

PRNE 36

Was condemned by the CNE Railway on 31 Dec 1912 for $500.00 scrap value. (See CNEW note 37) (PR&NE Class “A”) (To CNE (1st) #32)

PRNE 37

Was wrecked at Simsbury , CT on 28 Mar 1893 and shipped to BLW in Philadelphia, PA for major repairs in Dec 1894. Sold by the CNE Railway to Fitzhugh & Co. (locomotive broker) in May 1903 for $2800.00 and by them in Oct 1903 to the Weed Lumber Co.of  Weed, CA becoming their #3. Line became the California Northeastern RR (also known locally as the “CNE”) in Jul 1905 with the locomotive retaining the #3. This line became part of the Southern Pacific RR on 1 Sep 1910 with this locomotive becoming their (2nd) #2500 (Class C-6). Condemned on 31 Jul 1927 and scrapped in Sacremento, CA on 23 Sep 1927. (See CNEW note 38) (PR&NE Class “A”) (To CNE (1st) #33)

PRNE 38

BLW Class 10 21/40E-4. Shipped new from BLW in Philadelphia, PA on Monday, 4 Dec 1893 to Maybrook, NY. Was wrecked at Canton, CT on 23 Jun 1899. Cylinders were simpled to 20” X 24” by the CNE Railway in late 1904 or early 1905. Condemned on 1 July 1925. (To CNE #34 - #85)

PRNE 39

BLW Class 10 21/40E-5. Shipped new from BLW in Philadelphia, PA on Monday, 4 Dec 1893 to Maybrook, NY. Cylinders were simpled to 20” X 24” by the CNE Railway in late 1905 or early 1905. Was condemned in 1925. (To CNE (1st) #35 - #86)

PRNE 40

BLW Class 10 21/40E-6. Shipped new from BLW in Philadelphia, PA on Thursday, 7 Dec 1893 to Maybrook, NY. Cylinders were simpled to 20” X 24” by the CNE Railway in late 1905 or early 1905. Was condemned in June 1925. (To CNE (1st) #36 - #87)

PRNE 41

BLW Class 10 21/40E-7. Shipped new from BLW in Philadelphia, PA on Thursday, 7 Dec 1893 to Maybrook, NY. Cylinders were simpled to 20” X 24” by the CNE Railway in late 1905 or early 1905. Was condemned in June 1925. (To CNE (1st) #37 - #88)

PRNE 42

BLW Class 10 21/40E-8. Shipped new from BLW in Philadelphia, PA on Saturday, 9 Dec 1893 to Maybrook, NY. Cylinders were simpled to 20” X 24” by the CNE Railway in late 1905 or early 1905. Was condemned in June 1926. (To CNE (1st) #38 - #89)

PRNE 43

BLW Class 10 21/40E-9. Shipped new from BLW in Philadelphia, PA on Saturday, 9 Dec 1893 to Maybrook, NY. Cylinders were simpled to 20” X 24” by the CNE Railway in late 1905 or early 1905. Was condemned in June 1926. (To CNE (1st) #39 - #90)

PRNE 44

BLW Class 10 21/40E-10. Shipped new from BLW in Philadelphia, PA on Thursday, 14 Dec 1893 to Hartford, CT. Was sold by the CNE Railway on 4 Sep 1905 to the Fitzhugh – Luther & Co. (locomotive broker) for $3000.00 and apparently ended up on a railroad somewhere in the California / Nevada desert area. Final disposition unknown. (To CNE (1st) #40)

PRNE 45

Was turned over to the PR&NE RR upon the P&R Railway’s takeover of the CNE&W RR in Aug 1892. However, it very quickly disappeared from the roster and was either sold or scrapped shortly thereafter. (Was R&C #2 – (2nd) #1; H&CW #11; CNE&W (1st) #11 - #41)

PRNE 46

Was turned over to the PR&NE RR upon the P&R Railway’s takeover of the CNE&W RR in Aug 1892. However, it very quickly disappeared from the roster and was either sold or scrapped shortly thereafter. (Was R&C #4 – (2nd) #3; H&CW #13; CNE&W (1st) #13 - #43)

Central New England Railway

(1899 – 1927)

Locomotive Roster Notes

| 1899 Number | 1905-07 Renumber | 1913 Number | Name | Builder | Const # | Build Date | Delivery Date | Cylinder Size | Driver Diameter | Wheel Arrangement | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1st) 1 |  |  |  | H.K. Porter | 1075,76,or78 |  | 8or9/1889 | 10" x 14" | 22" | 04-2T | CNE 1 |
| (2nd) 1 | (2nd) 1 |  |  | Rogers L&M Co. | 4108 |  | 2/11/1889 | 19" x 24" | 56" | 2-6-0 | CNE 2 |
|  |  | (3rd) 1 |  | Alco/ Schtdy | 29989 |  | 7/1904 | 20" x 24" | 51" | 0-6-0 | CNE 3 |
| (1st) 2 |  |  |  | Baldwin L.W. | 9828 |  | 2/1889 | 19" x 24" | 54" | 2-6-0 | CNE 4 |
|  |  | (2nd) 2 |  | Alco/ Schtdy | 29990 |  | 7/1904 | 20" x 24" | 51" | 0-6-0 | CNE 5 |
|  | (1st) 3 |  |  | Danforth L&M | 804 |  | 3/30/1872 | 18" x 24" | 56" | 2-6-0 | CNE 6 |
|  |  | (2nd) 3 |  | Alco/ Schtdy | 42504 |  | 3/1907 | 20" x 24" | 51" | 0-6-0 | CNE 7 |
| (1st) 4 |  |  |  | Baldwin L.W. | 13857 |  | 12/1893 | 12+20" x 24" | 68" | 4-4-0 | zzzz |
| (1st) 4 |  |  |  | Baldwin L.W. | 13857 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 8 |
|  | (2nd) 4 |  |  | Baldwin L.W. | 3046 |  | 12/1872 | 20" x 24" | 56" | 4-6-0 | CNE 9 |
|  |  | (3rd) 4 |  | Alco/ Schtdy | 42744 |  | 4/1907 | 20" x 24" | 51" | 0-6-0 | CNE 10 |
| (1st) 5 |  |  | Norfolk | Rogers L&M Co. | *1969 | 10/30/1871 | 11/1871 | 16" x 24" | 60" | 4-4-0 | CNE 11 |
|  | (2nd) 5 |  |  | Dickson Man. | 280 |  | 1/16/1881 | 18" x 24" | 55.75" | 2-6-0 | CNE 12 |
|  |  | (3rd) 5 |  | Alco/ Schtdy | 48025 |  | 7/1910 | 20" x 24" | 51" | 0-6-0 | CNE 13 |
| (1st) 6 |  |  | Bloomfield | Rogers L&M Co. | *1838 |  | 1/9/1871 | 16" x 22" | 66" | 4-4-0 | CNE 14 |
|  |  | (2nd) 6 |  | Alco/ Schtdy | 48026 |  | 7/1910 | 20" x 24" | 51" | 0-6-0 | CNE 15 |
| (1st) 7 |  |  |  | Baldwin L.W. | 13858 |  | 12/1893 | 12+20"x24" | 68" | 4-4-0 | zzzz |
| (1st) 7 |  |  |  | Baldwin L.W. | 13858 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 16 |
|  |  | (2nd) 7 |  | Alco/ Schtdy | 29989 |  | 7/1904 | 20" x 24" | 51" | 0-6-0 | CNE 17 |
| 8 |  |  |  | Baldwin L.W. | 13859 |  | 12/1893 | 12+20"x24" | 68" | 4-4-0 | zzzz |
| 8 |  |  |  | Baldwin L.W. | 13859 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 18 |
| (1st) 10 |  |  | Tariffville | Rogers L&M Co. | 2429 |  | 12/10/1871 | 17" x 22" | 60" | 4-4-0 | CNE 19 |
|  |  | (2nd) 10 |  | Alco/ Schtdy | 52813 |  | 3/1/1913 | 22" x 28" | 51" | 0-8-0 | CNE 20 |
| (1st) 11 | (1st) 11 |  |  | Baldwin L.W. | 12413 |  | 1/11/1892 | 21" x 24" | 62" | 4-6-0 | CNE 21 |
|  |  | (2nd) 11 |  | Alco/ Schtdy | 52814 |  | 3/1/1913 | 22" x 28" | 51" | 0-8-0 | CNE 22 |
| (1st) 12 | (1st) 12 |  |  | Baldwin L.W. | 12412 |  | 1/9/1892 | 21" x 24" | 62" | 4-6-0 | CNE 23 |
|  |  | (2nd) 12 |  | Alco/ Schtdy | 52815 |  | 3/1/1913 | 22" x 28" | 51" | 0-8-0 | CNE 24 |
| (1st) 13 | (1st) 13 |  |  | Baldwin L.W. | 12390 |  | 12/31/1891 | 21" x 24" | 62" | 4-6-0 | CNE 25 |
|  |  | (2nd) 13 |  | Alco/ Schtdy | 63400 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 26 |
| (1st) 14 | (1st) 14 |  |  | Baldwin L.W. | 12391 |  | 12/31/1891 | 21" x 24" | 62" | 4-6-0 | CNE 27 |
|  |  | (2nd) 14 |  | Alco/ Schtdy | 63401 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 28 |
| (1st) 15 |  |  |  | Rogers L&M Co. | 3052 |  | 8/7/1882 | 18" x 24" | 60" | 4-4-0 | CNE 29 |
|  |  | (2nd) 15 |  | Alco/ Schtdy | 63402 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 30 |
| (1st) 16 |  |  |  | Rogers L&M Co. | 3096 |  | 9/30/1882 | 18" x 24" | 62" | 4-4-0 | CNE 31 |
|  |  | (2nd) 16 |  | Alco/ Schtdy | 63403 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 32 |
| (1st) 17 |  |  |  | Rogers L&M Co. | 3299 |  | 7/6/1883 | 18" x 24" | 62" | 4-4-0 | CNE 33 |
|  |  | (2nd) 17 |  | Alco/ Schtdy | 63404 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 34 |
| (1st) 18 |  |  |  | Rogers L&M Co. | 3346 |  | 9/10/1883 | 18" x 24" | 62" | 4-4-0 | CNE 35 |
|  |  | (2nd) 18 |  | Alco/ Schtdy | 63405 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 36 |
| (1st) 19 |  |  |  | Baldwin L.W. | 9572 |  | 10/1888 | 18" x 24" | 63" | 4-4-0 | CNE 37 |
|  |  | (2nd) 19 |  | Alco/ Schtdy | 63406 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 38 |
| (1st) 20 |  |  |  | Baldwin L.W. | 11208 |  | 9/23/1890 | 18" x 24" | 63" | 4-4-0 | CNE 39 |
|  |  | (2nd) 20 |  | Alco/ Schtdy | 63407 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 40 |
| (1st) 21 |  |  |  | Baldwin L.W. | 9571 |  | 10/1888 | 18" x 24" | 63" | 4-4-0 | CNE 41 |
|  |  | (2nd) 21 |  | Alco/ Schtdy | 63408 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 42 |
| (1st) 22 |  |  |  | Rogers L&M Co. | 4108 |  | 2/11/1889 | 19" x 24" | 56" | 2-6-0 | CNE 43 |
|  |  | (2nd) 22 |  | Alco/ Schtdy | 63409 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 44 |
| (1st) 23 |  |  |  | Baldwin L.W. | 9828 |  | 2/1889 | 19" x 24" | 54" | 2-6-0 | CNE 45 |
|  |  | (2nd) 23 |  | Alco/ Schtdy | 63410 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 46 |
| (1st) 24 |  |  |  | Baldwin L.W. | 9863 |  | 3/1889 | 20" x 24" | 50" | 2-8-0 | CNE 47 |
|  |  | (2nd) 24 |  | Alco/ Schtdy | 63411 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 48 |
| (1st) 25 | (1st) 25 |  |  | Baldwin L.W. | 9860 |  | 3/1889 | 20" x 24" | 50" | 2-8-0 | CNE 49 |
|  |  | (2nd) 25 |  | Baldwin L.W. | 12413 |  | 1/1892 | 21" x 24" | 62" | 4-6-0 | CNE 50 |
|  |  | (3rd) 25 |  | Alco/ Schtdy | 63412 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 51 |
| (1st) 26 | (1st) 26 |  |  | Baldwin L.W. | 9861 |  | 3/1889 | 20" x 24" | 50" | 2-8-0 | CNE 52 |
|  |  | (2nd) 26 |  | Baldwin L.W. | 12412 |  | 1/1892 | 21" x 24" | 62" | 4-6-0 | CNE 53 |
|  |  | (3rd) 26 |  | Alco/ Schtdy | 63413 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 54 |
| (1st) 27 |  |  |  | Baldwin L.W. | 11217 |  | 9/1890 | 20" x 24" | 50" | 2-8-0 | CNE 55 |
|  |  | (2nd) 27 |  | Baldwin L.W. | 12390 |  | 12/1891 | 21" x 24" | 62" | 4-6-0 | CNE 56 |
|  |  | (3rd) 27 |  | Alco/ Schtdy | 63414 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 57 |
| (1st) 28 |  |  |  | Baldwin L.W. | 11214 |  | 9/1890 | 20" x 24" | 50" | 2-8-0 | CNE 58 |
|  |  | (2nd) 28 |  | Baldwin L.W. | 12391 |  | 12/1891 | 21" x 24" | 62" | 4-6-0 | CNE 59 |
|  |  | (3rd) 28 |  | Alco/ Schtdy | 63457 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 60 |
| (1st) 29 |  |  |  | Baldwin L.W. | 10839 |  | 4/1890 | 20" x 24" | 50" | 2-8-0 | CNE 61 |
|  |  | (2nd) 29 |  | Alco/ Schtdy | 63458 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 62 |
| (1st) 30 |  |  |  | Baldwin L.W. | 10843 |  | 4/1890 | 20" x 24" | 50" | 2-8-0 | CNE 63 |
|  |  | (2nd) 30 |  | Alco/ Schtdy | 63459 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 64 |
| (1st) 31 | (1st) 31 |  |  | Baldwin L.W. | 11476 |  | 12/1890 | 20" x 24" | 50" | 2-8-0 | CNE 65 |
|  |  | (2nd) 31 |  | Baldwin L.W. | 9572 |  | 10/1888 | 18" x 24" | 63" | 4-4-0 | CNE 66 |
|  |  | (3rd) 31 |  | Alco/ Schtdy | 63460 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 67 |
| (1st) 32 |  |  |  | Baldwin L.W. | 11477 |  | 12/1890 | 20" x 24" | 50" | 2-8-0 | CNE 68 |
|  |  | (2nd) 32 |  | Baldwin L.W. | 11208 |  | 9/1890 | 18" x 24" | 63" | 4-4-0 | CNE 69 |
|  |  | (3rd) 32 |  | Alco/ Schtdy | 63461 |  | 8/1922 | 25" x 28" | 51" | 0-8-0 | CNE 70 |
| (1st) 33 |  |  |  | Baldwin L.W. | 11478 |  | 12/1890 | 20" x 24" | 50" | 2-8-0 | CNE 71 |
|  |  | (2nd) 33 |  | Baldwin L.W. | 9571 |  | 10/1888 | 18" x 24" | 63" | 4-4-0 | CNE 72 |
| 34 | 34 |  |  | Baldwin L.W. | 13847 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 73 |
| (1st) 35 | (1st) 35 |  |  | Baldwin L.W. | 13848 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 74 |
|  |  | (2nd) 35 |  | Rogers L&M Co. | 3096 |  | 9/30/1882 | 18" x 24" | 62" | 4-4-0 | CNE 75 |
| (1st) 36 | (1st) 36 |  |  | Baldwin L.W. | 13849 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 76 |
|  |  | (2nd) 36 |  | Rogers L&M Co. | 3299 |  | 7/6/1883 | 18" x 24" | 62" | 4-4-0 | CNE 77 |
| (1st) 37 | (1st) 37 |  |  | Baldwin L.W. | 13850 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 78 |
|  |  | (2nd) 37 |  | Rogers L&M Co. | 3346 |  | 9/10/1883 | 18" x 24" | 62" | 4-4-0 | CNE 79 |
| (1st) 38 | (1st) 38 |  |  | Baldwin L.W. | 13851 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 80 |
|  |  | (2nd) 38 |  | Baldwin L.W. | 14276 |  | 4/1895 | 18" x 24" | 69" | 4-4-0 | CNE 81 |
| (1st) 39 | (1st) 39 |  |  | Baldwin L.W. | 13852 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 82 |
|  |  | (2nd) 39 |  | Baldwin L.W. | 21016 |  | 9/1902 | 19" x 24" | 69" | 4-4-0 | CNE 83 |
| (1st) 40 |  |  |  | Baldwin L.W. | 13853 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 84 |
|  |  | (2nd) 40 |  | Baldwin L.W. | 19461 |  | 8/1901 | 18" x 24" | 69" | 4-4-2 | CNE 85 |
| (1st) 41 | (1st) 41 |  |  | Alco/ Schtdy | 29991 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 86 |
|  |  | (2nd) 41 |  | Baldwin L.W. | 13857 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 87 |
| (1st) 42 | (1st) 42 |  |  | Alco/ Schtdy | 29992 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 88 |
|  |  | (2nd) 42 |  | Baldwin L.W. | 13859 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 89 |
| (1st) 43 | (1st) 43 |  |  | Alco/ Schtdy | 29993 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 90 |
|  |  | (2nd) 43 |  | Baldwin L.W. | 13858 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 91 |
| 44 | 44 |  |  | Alco/ Schtdy | 29994 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 92 |
| (1st) 45 | (1st) 45 |  |  | Alco/ Schtdy | 29995 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 93 |
|  |  | (2nd) 45 |  | Norwood Shops | ? |  | 1903 | 18" x 26" | 69" | 4-4-0 | CNE 94 |
|  | (1st) 46 |  |  | Alco/ Rogers | 39036 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 95 |
|  |  | (2nd) 46 |  | N.H. Shops | 11438 |  | 1903 | 18" x 26" | 69" | 4-4-0 | CNE 96 |
|  | (1st) 47 |  |  | Alco/ Rogers | 39037 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 97 |
|  |  | (2nd) 47 |  | Baldwin L.W. | 12413 |  | 1/1892 | 21" x 24" | 62" | 4-6-0 | CNE 98 |
|  | (1st) 48 |  |  | Alco/ Rogers | 39038 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 99 |
|  |  | (2nd) 48 |  | Baldwin L.W. | 12391 |  | 12/1891 | 21" x 24" | 62" | 4-6-0 | CNE 100 |
|  | 49 |  |  | Alco/ Rogers | 39039 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 101 |
|  | (1st) 50 |  |  | Alco/ Rogers | 39040 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 102 |
|  |  | (2nd) 50 |  | Alco/ Schtdy | 46276 |  | 7/1909 | 20" x 26" | 69" | 4-6-0 | CNE 103 |
|  | (1st) 51 |  |  | Alco/ Rogers | 39041 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 104 |
|  |  | (2nd) 51 |  | Alco/ Schtdy | 46277 |  | 7/1909 | 20" x 26" | 69" | 4-6-0 | CNE 105 |
|  | (1st) 52 |  |  | Alco/ Rogers | 39042 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 106 |
|  |  | (2nd) 52 |  | Alco/ Schtdy | 46278 |  | 7/1909 | 20" x 26" | 69" | 4-6-0 | CNE 107 |
|  | 53 |  |  | Alco/ Rogers | 39043 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 108 |
|  | 54 |  |  | Alco/ Rogers | 39044 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 109 |
|  | 55 |  |  | Alco/ Rogers | 39045 |  | 1/1906 | 20" x 24" | 50" | 2-8-0 | CNE 110 |
|  | 56 |  |  | Alco/ Rogers | 44327 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 111 |
|  | 57 |  |  | Alco/ Rogers | 44328 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 112 |
|  | 58 |  |  | Alco/ Rogers | 44329 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 113 |
|  | 59 |  |  | Alco/ Rogers | 44330 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 114 |
|  | 60 |  |  | Alco/ Rogers | 44331 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 115 |
|  | 61 |  |  | Alco/ Rogers | 44332 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 116 |
| 62 Thru 74 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 75 |  | Baldwin L.W. | 9860 |  | 3/1889 | 20" x 24" | 50" | 2-8-0 | CNE 117 |
|  |  | ?76? |  | Baldwin L.W. | 11476 |  | 12/1890 | 20" x 24" | 50" | 2-8-0 | CNE 118 |
|  |  | 77 |  | Baldwin L.W. | 9828 |  | 2/1889 | 19" x 24" | 54" | 2-6-0 | CNE 119 |
| 79 Thru 84 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 85 |  | Baldwin L.W. | 13847 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 120 |
|  |  | 86 |  | Baldwin L.W. | 13848 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 121 |
|  |  | 87 |  | Baldwin L.W. | 13849 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 122 |
|  |  | 88 |  | Baldwin L.W. | 13850 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 123 |
|  |  | 89 |  | Baldwin L.W. | 13851 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 124 |
|  |  | 90 |  | Baldwin L.W. | 13852 |  | 11/1893 | 20" x 24" | 51" | 2-8-0 | CNE 125 |
| 91 Thru 99 Vacant |  |  |  |  |  |  |  |  |  |  |  |
| (1st) 100 | (1st) 100 |  |  | Alco/ Schtdy | 29989 |  | 7/1904 | 20" x 24" | 51" | 0-6-0 | CNE 126 |
|  |  | (2nd) 100 |  | Alco/ Schtdy | 29991 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 127 |
| (1st) 101 | (1st) 101 |  |  | Alco/ Schtdy | 29990 |  | 7/1904 | 20" x 24" | 51" | 0-6-0 | CNE 128 |
|  |  | (2nd) 101 |  | Alco/ Schtdy | 29992 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 129 |
|  | (1st) 102 |  |  | Alco/ Schtdy | 42504 |  | 3/1907 | 20" x 24" | 51" | 0-6-0 | CNE 130 |
|  |  | (2nd) 102 |  | Alco/ Schtdy | 29993 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 131 |
|  | (1st) 103 |  |  | Alco/ Schtdy | 42744 |  | 4/1907 | 20" x 24" | 51" | 0-6-0 | CNE 132 |
|  |  | (2nd) 103 |  | Alco/ Schtdy | 29994 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 133 |
|  | (1st) 104 |  |  | Baldwin L.W. | 9828 |  | 2/1889 | 19" x 24" | 54" | 2-6-0 | CNE 134 |
|  | (2nd) 104 |  |  | Alco/ Schtdy | 48025 |  | 7/1910 | 20" x 24" | 51" | 0-6-0 | CNE 135 |
|  |  | (3rd) 104 |  | Alco/ Schtdy | 49995 |  | 7/1904 | 20" x 24" | 50" | 2-8-0 | CNE 136 |
|  | (1st) 105 |  |  | Alco/ Schtdy | 48026 |  | 7/1910 | 20" x 24" | 51" | 0-6-0 | CNE 137 |
|  |  | (2nd) 105 |  | Alco/ Rogers | 39036 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 138 |
|  | (1st) 106 |  |  | Baldwin L.W. | 9828 |  | 2/1889 | 19" x 24" | 54" | 2-6-0 | CNE 139 |
|  |  | (2nd) 106 |  | Alco/ Rogers | 39037 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 140 |
|  |  | ((1st) 107) |  | Alco/ Schtdy | 52813 |  | 2/1913 | 20" x 24" | 51" | 0-6-0 | CNE 141 |
|  |  | (2nd) 107 |  | Alco/ Rogers | 39038 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 142 |
|  |  | ((1st) 108) |  | Alco/ Schtdy | 52814 |  | 2/1913 | 22" x 28" | 51" | 0-8-0 | CNE 143 |
|  |  | (2nd) 108 |  | Alco/ Rogers | 39039 |  | 11/1905 | 20" x 24" | 50" | 2-8-0 | CNE 144 |
|  |  | ((1st) 109) |  | Alco/ Schtdy | 52815 |  | 2/1913 | 22" x 28" | 51" | 0-8-0 | CNE 145 |
|  |  | (2nd) 109 |  | Alco/ Rogers | 39040 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 146 |
|  |  | 110 |  | Alco/ Rogers | 39041 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 147 |
|  |  | 111 |  | Alco/ Rogers | 39042 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 148 |
|  |  | 112 |  | Alco/ Rogers | 39043 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 149 |
|  |  | 113 |  | Alco/ Rogers | 39044 |  | 12/1905 | 20" x 24" | 50" | 2-8-0 | CNE 150 |
|  |  | 114 |  | Alco/ Rogers | 39045 |  | 1/1906 | 20" x 24" | 50" | 2-8-0 | CNE 151 |
|  |  | 115 |  | Alco/ Rogers | 44327 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 152 |
|  |  | 116 |  | Alco/ Rogers | 44328 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 153 |
|  |  | 117 |  | Alco/ Rogers | 44329 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 154 |
|  |  | 118 |  | Alco/ Rogers | 44330 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 155 |
|  |  | 119 |  | Alco/ Rogers | 44331 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 156 |
|  |  | 120 |  | Alco/ Rogers | 44332 |  | 11/1907 | 20" x 24" | 50" | 2-8-0 | CNE 157 |
| 121 Thru 124 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 125 |  | Alco/ Brooks | 49098 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 158 |
|  |  | 126 |  | Alco/ Brooks | 49099 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 159 |
|  |  | 127 |  | Alco/ Brooks | 49100 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 160 |
|  |  | 128 |  | Alco/ Brooks | 49101 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 161 |
|  |  | 129 |  | Alco/ Brooks | 49102 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 162 |
|  |  | 130 |  | Alco/ Brooks | 49103 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 163 |
|  |  | 131 |  | Rhode Island | 2896 |  | 6/1893 | 20" x 26" | 63" | 2-6-0 | CNE 164 |
| 132 Thru 149 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 150 |  | Alco/ Schtdy | 51717 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 165 |
|  |  | 151 |  | Alco/ Schtdy | 51718 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 166 |
|  |  | 152 |  | Alco/ Schtdy | 51719 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 167 |
|  |  | 153 |  | Alco/ Schtdy | 51720 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 168 |
|  |  | 154 |  | Alco/ Schtdy | 51721 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 169 |
|  |  | 155 |  | Alco/ Schtdy | 51722 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 170 |
|  |  | 156 |  | Alco/ Schtdy | 51723 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 171 |
|  |  | 157 |  | Alco/ Schtdy | 51724 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 172 |
|  |  | 158 |  | Alco/ Schtdy | 51725 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 173 |
|  |  | 159 |  | Alco/ Schtdy | 51726 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 174 |
|  |  | 160 |  | Alco/ Schtdy | 51727 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 175 |
|  |  | 161 |  | Alco/ Schtdy | 51728 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 176 |
|  |  | 162 |  | Alco/ Schtdy | 51729 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 177 |
|  |  | 163 |  | Alco/ Schtdy | 51730 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 178 |
|  |  | 164 |  | Alco/ Schtdy | 51731 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 179 |
| 165 Thru 179 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 180 |  | Alco/ Schtdy | 55725 |  | 5/1916 | 26" x 32" | 63" | 2-8-2 | CNE 180 |
|  |  | 181 |  | Alco/ Schtdy | 55726 |  | 5/1916 | 26" x 32" | 63" | 2-8-2 | CNE 181 |
|  |  | 182 |  | Alco/ Schtdy | 55727 |  | 5/1916 | 26" x 32" | 63" | 2-8-2 | CNE 182 |
| 183 Thru 199 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  | 200 |  |  | Baldwin L.W. | 13857 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 183 |
|  | 201 |  |  | Baldwin L.W. | 13858 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 184 |
|  | 202 |  |  | Baldwin L.W. | 13859 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 185 |
|  | 203 |  |  | Cooke L&M | 2263 |  | 5/1893 | 17" x 24" | 60" | 4-4-0 | CNE 186 |
|  | 204 |  |  | Rogers L&M Co. | 4848 |  | 4/1893 | 17" x 24" | 62" | 4-4-0 | CNE 187 |
|  | 205 |  |  | Rogers L&M Co. | 3585 |  | 10/1885 | 17" x 24" | 62" | 4-4-0 | CNE 188 |
|  | 206 |  |  | Schenectady | 1716 |  | 5/1883 | 17" x 24" | 64" | 4-4-0 | CNE 189 |
| 207 Thru 208 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  | 209 |  |  | Portland Co. | 411 |  | 12/1881 | 17" x 24" | 61" | 4-4-0 | CNE 190 |
|  | 210 |  | Tariffville | Rogers L&M Co. | 2429 |  | 12/1875 | 17" x 22" | 60" | 4-4-0 | CNE 191 |
|  | 211 |  |  | Rogers L&M Co. | 3052 |  | 8/1882 | 18" x 24" | 60" | 4-4-0 | CNE 192 |
|  | 212 |  |  | Grant L.W. | 700 |  | 4/1870 | 17" x 24" | 60" | 4-4-0 | CNE 193 |
|  | 213 |  |  | Schenectady | 616 |  | 3/1870 | 16" x 24" | 60" | 4-4-0 | CNE 194 |
|  | 214 |  |  | Brooks L.W. | 188 |  | 6/1873 | 16" x 24" | 62" | 4-4-0 | CNE 195 |
|  | 215 |  |  | Brooks L.W. | 193 |  | 7/1873 | 16" x 24" | 62" | 4-4-0 | CNE 196 |
|  | 216 |  |  | Brooks L.W. | 190 |  | 6/1873 | 16" x 24" | 62" | 4-4-0 |  |
| 217 Thru 219 Vacant |  |  |  |  |  |  |  |  |  |  | CNE 197 |
|  | 220 |  |  | Rogers L&M Co. | 3096 |  | 9/1882 | 18" x 24" | 62" | 4-4-0 | CNE 198 |
|  | 221 |  |  | Rogers L&M Co. | 3299 |  | 7/1883 | 18" x 24" | 62" | 4-4-0 | CNE 199 |
|  | 222 |  |  | Rogers L&M Co. | 3346 |  | 9/1883 | 18" x 24" | 62" | 4-4-0 | CNE 200 |
|  | 223 |  |  | Baldwin L.W. | 9572 |  | 10/1888 | 18" x 24" | 63" | 4-4-0 | CNE 201 |
|  | 224 |  |  | Baldwin L.W. | 11208 |  | 9/1890 | 18" x 24" | 63" | 4-4-0 | CNE 202 |
|  | 225 |  |  | Baldwin L.W. | 9571 |  | 10/1888 | 18" x 24" | 63" | 4-4-0 | CNE 203 |
|  | 226 |  |  | Baldwin L.W. | 14276 |  | 4/1895 | 18" x 24" | 69" | 4-4-0 | CNE 204 |
|  | 227 |  |  | Baldwin L.W. | 21016 |  | 9/1902 | 18" x 24" | 69" | 4-4-0 | CNE 205 |
|  | 228 |  |  | Baldwin L.W. | 19461 |  | 8/1901 | 18" x 24" | 69" | 4-4-0 | CNE 206 |
|  | 229 |  |  | Baldwin L.W. | 13857 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 207 |
|  | 230 |  |  | Baldwin L.W. | 13859 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 208 |
|  | 231 |  |  | Baldwin L.W. | 13858 |  | 12/1893 | 18" x 24" | 69" | 4-4-0 | CNE 209 |
| 232 Thru 299 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  | 300 |  |  | Alco/ Schtdy | 46276 |  | 7/1909 | 20" x 26" | 69" | 4-6-0 | CNE 210 |
|  | 301 |  |  | Alco/ Schtdy | 46277 |  | 7/1909 | 20" x 26" | 69" | 4-6-0 | CNE 211 |
|  | 302 |  |  | Alco/ Schtdy | 46278 |  | 7/1909 | 20" x 26" | 69" | 4-6-0 | CNE 212 |
| 303 Thru 399 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  | 400 |  |  | Alco/ Brooks | 49098 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 213 |
|  | 401 |  |  | Alco/ Brooks | 49099 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 214 |
|  | 402 |  |  | Alco/ Brooks | 49100 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 215 |
|  | 403 |  |  | Alco/ Brooks | 49101 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 216 |
|  | 404 |  |  | Alco/ Brooks | 49102 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 217 |
|  | 405 |  |  | Alco/ Brooks | 49103 |  | 11/1910 | 20" x 28" | 63" | 2-6-0 | CNE 218 |
| 406 Thru 499 Vacant |  |  |  |  |  |  |  |  |  |  |  |
|  | 500 |  |  | Alco/ Schtdy | 51717 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 219 |
|  | 501 |  |  | Alco/ Schtdy | 51718 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 220 |
|  | 502 |  |  | Alco/ Schtdy | 51719 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 221 |
|  | 503 |  |  | Alco/ Schtdy | 51720 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 222 |
|  | 504 |  |  | Alco/ Schtdy | 51721 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 223 |
|  | 505 |  |  | Alco/ Schtdy | 51722 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 224 |
|  | 506 |  |  | Alco/ Schtdy | 51723 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 225 |
|  | 507 |  |  | Alco/ Schtdy | 51724 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 226 |
|  | 508 |  |  | Alco/ Schtdy | 51725 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 227 |
|  | 509 |  |  | Alco/ Schtdy | 51726 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 228 |
|  | 510 |  |  | Alco/ Schtdy | 51727 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 229 |
|  | 511 |  |  | Alco/ Schtdy | 51728 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 230 |
|  | 512 |  |  | Alco/ Schtdy | 51729 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 231 |
|  | 513 |  |  | Alco/ Schtdy | 51730 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 232 |
|  | 514 |  |  | Alco/ Schtdy | 51731 |  | 9/1912 | 24" x 32" | 63" | 2-8-0 | CNE 233 |

( ) = Locomotive numbers bracketed by “( )” marks indicate those built in early 1913 just prior to the major renumbering done later that same year.

* =  Indicates unofficial construction number as Rogers apparently used none between Jun 1856 and Nov 1872. (R&LHS Bulletin #167 – Fall, 1992)

@ = Originally thought that the 200 series renumbering of the CNE’s passenger locomotives was not begun until sometime in 1907 after both the ND&C and the P&E locomotives were absorbed in to the CNE roster; however, train orders that were dated in Dec 1904 indicate that some of the locomotives were already carrying the 200 series numbers by that date.

= Indicates probable delivery date to the CNE rather than the builders completion date.

CNE 1

Was used on the Poughkeepsie Bridge trolley shuttle service. Also used in work train service on EG&S in 1902. Disposed of in May 1903 to an F.M. Hicks (locomotive broker in Chicago, IL) for $800.00. Final disposition unknown. (See PRNE note 2) (Was PR&NE (2nd) #1)

CNE 2

Renumbered to (2nd) #1 sometime after May 1903 when Porter Dummy locomotive #1 was sold. Condemned on 31 Dec 1911 for scrap value of $300.00. (See HCW note 19) (Was H&CW 19; CNE&W (1st) #19 – (2nd) #22; PR&NE #22; CNE (1st) #22)

CNE 3

Sold to the NH RR in Nov 1904 for $10,250.00 and became their #2400. Sold back to the CNE in Nov 1905 for $9367.34 becoming (1st) #100 again. Became CNE (3rd) #1 in late 1913 and (2nd) #7 in Aug 1925. Possible candidate for NH RR #2350 but was condemned in Sep 1927. (CNE Class Y-1/ NH Class T-3)

CNE 4

Was renumbered from (1st) #23 to (1st) #2 by the CNE in late 1903 or early 1904. Was the replacement locomotive for the Hartford, CT switcher after ex-R&C 0-4-2T was scrapped. Condemned on 1 July 1914 for scrap value of $350.00 (See HC RR note 3) (Class Y-1) (Was HC #3; CNE&W #23; PR$NE #23 - #1272 - #23; To CNE (1st) #104 – (1st) #106 - #77)

CNE 5

Sold to the NH RR in Nov 1904 for $10,250.00 and became their #2401. Sold back to the CNE in Nov 1905 for $9341.14 and becoming (1st) #101 again. Became CNE (2nd) #2 in late 1913 and NH RR #2351 in Sep 1927. Was condemned on 31 Jan 1935 and scrapped at the Readville, MA shops on 28 Mar 1935. (CNE Class Y-1/ NH Class T-3)

CNE 6

Was condemned in Mar 1906 and sold to W.B. Seymore Co. (scrap dealer) for $626.25. (See ND&C note 11) (Was D&C #5; NYB&M #15; ND&C (1st) #8 – (2nd) #3)

CNE 7

Was CNE (1st) #102 to late 1913. Became NH RR #2352 in Sep 1927. Sold to the Lathrop & Shea Co. on 21 May 1930 and became their #510. Final disposition unknown. (CNE Class Y-1/ NH Class T-3)

CNE 8

Cylinders simpled to 18” X 24” and drivers extended to 69” during rebuilding in Hartford, CT shops in 1908. Was condemned in Nov 1926. (See PR&NE note 6) (Class P-2) (Was PR&NE (2nd) #4; To CNE #200 - #229 – (2nd) #41)

CNE 9

Was condemned on 21 July 1908 for value of $240.00 and scrapped soon after. (See P&E Ry note 8) (Was P&E Ry (2nd) #4)

CNE 10

Was CNE (1st) #103 to late 1913. Became NH RR #2353 in Sep 1927. Was rebuilt to an 0-6-0T in 1938. Was last existing CNE Railway locomotive. Condemned in May 1953. (CNE Class Y-1/ NH Class T-3)

CNE 11

Was in a condemned state in March 1899 but was repaired and returned to service for a short period of time on the Poughkeepsie Bridge trolley shuttle service. Sold to American Iron & Steel Co. of Lebanon, PA for scrap and was shipped to them on 16 Jan 1901. (See CW note 6) (Was CW #5; H&CW #5; CNE&W #5; PR&NE #5 - #1254 - #5)

CNE 12

Was condemned in April 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $165.00. (See P&E Ry note 11) (Was P&E Ry (3rd) #5)

CNE 13

Was CNE (2nd) #104 until 15 Nov 1913. Became NH RR #2354 in Sep 1927. Was condemned at Midway, CT on 28 Feb 1930 and scrapped at the Readville, MA shops on 5 Sep 1935. (CNE Class Y-1/ NH Class T-3)

CNE 14

Was condemned on 9 Mar 1899 and shipped to the Pennsylvania Bolt & Nut Co. of Lebanon, PA on 18 Mar 1899 for scrapping. (See CW note 7) (Was CW #6; H&CW #6; CNE&W #6; PR&NE #6 - #1255 - #6)

CNE 15

Was CNE (1st) #105 until 15 Nov 1913. Became NH RR #2355 in Sep 1927. Was condemned on 31 Dec 1935 and scrapped at the Readville, MA shops on 30 Jun 1936. (CNE Class Y-1/ NH Class T-3)

CNE 16

Cylinders simpled to 18” X 24” and drivers extended to 69” during rebuilding in Hartford, CT shops in 1911. Was condemned on 10 Oct 1926. (See PR&NE note 10) (Class P-2) (Was PR&NE (2nd) #7; To CNE #201 - #231 – (2nd) #43)

CNE 17

Renumbered on 1 Aug 1925 from (3rd) #1 to (2nd) #7. Condemned in Sep 1927 prior to becoming a New Haven RR Locomotive. (Class Y-1)

CNE 18

Cylinders simpled to 18” X 24” and drivers extended to 69” during rebuilding in Hartford, CT shops in 1909. Was condemned in Nov 1925. (See PR&NE note 12) (Class P-2) (Was PR&NE (2nd) #8; To CNE #202 - #230 – (2nd) #42)

CNE 19

Was rebuilt at the Hartford, CT shops in Feb 1902. Sold on 2 Nov 1906 to U.I. Harris (scrap dealer) of Hartford, CT for $625.00. (See CW note 11) (Class P-2) (Was CW #10; H&CW #10; CNE&W #10; PR&NE #10; To CNE #210)

CNE 20

Was CNE (1st) #107 from 1 Mar 1913 until 15 Nov 1913 only. To NH RR #10 in Sep 1927. Condemned in Dec 1946. (CNE / NH Class Y-2)

CNE 21

Was condemned in Jul 1925. (See CNE&W note 12) (CNE / NH Class Y-2)

(Was CNE&W (2nd) #11; PR&NE #11 - #1260 - #11; To CNE (2nd) #25 – (2nd) #47)

CNE 22

Was CNE (1st) #108 from 1 Mar 1913 until 15 Nov 1913 only. To NH RR #11 in Sep 1927. Condemned in Jan 1946. (CNE / NH Class Y-2)

CNE 23

Was condemned on 31 Dec 1915 for scrap value of $300.00. (See CNE&W note 14) (Class P-4) (Was CNE&W (2nd) #12; PR&NE #12 - #1261 - #12; To CNE (2nd) #26)

CNE 24

Was CNE (1st) #109 from 1 Mar 1913 until 15 Nov 1913 only. To NH RR #12 in Sep 1927. Condemned in July 1948. (CNE / NH Class Y-2)

CNE 25

Was condemned on 30 Apr 1915 for scrap value of $350.00. (See CNE&W note 16) (Class P-4) (Was CNE&W (2nd) #13; PR&NE #13 - #1262 - #13; To CNE (2nd) #27)

CNE 26

To NH #3420 in Sep 1927. Condemned Mar 1952. (CNE / NH Class Y-3)

CNE 27

Condemned in Apr 1926. (See CNE&W note 17) (Class P-4) (Was CNE&W (2nd) #14; PR&NE #14 - #1263 - #14; To CNE (2nd) #28 – (2nd) #48)

CNE 28

To NH #3421 in Sep 1927. Condemned Sep or Oct 1951. (CNE / NH Class Y-3)

CNE 29

Was used primarily on the Rhinecliff Branch due to it’s light weight. Was condemned in Apr 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $200.00. (See H&CW note 15) (Class P-2) (Was H&CW #15; CNE&W #15; PR&NE #15 - #1264 - #15; To CNE #211)

CNE 30

To NH #3422 in Sep 1927. Condemned in Nov 1951. (CNE / NH Class Y-3)

CNE 31

Was rebuilt with new BLW boiler in July 1900 at the Hartford, CT shops at a cost of $5231.25. Was condemned on 20 Nov 1919. (See H&CW note 16) (Class P-2) (Was H&CW #16; CNE&W #16; PR&NE #16 - #1265 - #16; To CNE  #220 – (2nd) #35)

CNE 32

To NH #3423 in Sep 1927. Condemned in Jan 1949. (CNE / NH Class Y-3)

CNE 33

Was reboilered in the Hartford, CT shops in Apr 1901 at a cost of $5650.54. Was condemned in Oct 1925. (See H&CW note 17) (Class P-2) (Was H&CW #17; CNE&W #17; PR&NE #17 - #1266 - #17; To CNE #221 – (2nd) #36)

CNE 34

To NH #3424 in Sep 1927. Condemned in Jan 1949. (CNE / NH Class Y-3)

CNE 35

Cost new $9,572.00. Was reboilered and fitted with extended saddle in the Hartford, CT shops in Jul 1901at a cost of $5061.02. Was condemned in July 1926. (See H&CW note 18) (Class P-2) (Was H&CW #18; CNE&W #18; PR&NE #18 - #1267 - #18; To CNE #222 – (2nd) #37)

CNE 36

To NH #3425 in Sep 1927. Condemned in Jan 1949. (CNE / NH Class Y-3)

CNE 37

Was condemned on 31 Dec 1915 for scrap value of $300.00. (See HC note 2) (Class P-3) (Was HC #2; CNE&W (1st) #22 – (2nd) #19; PR&NE #19 - #1268 - #19; To CNE #223 – (2nd) #31)

CNE 38

To NH #3426 in Sep 1927. Condemned in Nov 1951. (CNE / NH Class Y-3)

CNE 39

Condemned on 30 Apr 1915 for $250.00 scrap value. (See CNE&W note 24) (Class P-3) (Was CNE&W #20; PR&NE #20 - #1269 - #20; To CNE #224 – (2nd) #32)

CNE 40

To NH #3427 in Sep 1927. Condemned in Jun 1950. (CNE / NH Class Y-3)

CNE 41

Was condemned on 30 Sep 1919 for $400.00 scrap value. (See HC RR note 1) (Class P-3) (Was HC #1; CNE&W (1st) #21; PR&NE #21 - #1270 - #21; To CNE #225 – (2nd) #33)

CNE 42

To NH #3428 in Sep 1927. Condemned in Sep 1950. (CNE / NH Class Y-3)

CNE 43

Was condemned on 31 Dec 1911 for value of $300.00 and was scrapped soon after. (See H&CW note 19) (Was H&CW #19; CNE&W (1st) #19 – (2nd) #22; PR&NE #22; To CNE (2nd) #1)

CNE 44

To NH #3429 in Sep 1927. Condemned in Oct 1950. (CNE / NH Class Y-3)

CNE 45

Condemned on 1 July 1914 for $350.00 scrap value. (See HC RR note 3) (Class Y-1) (Was HC #3; CNE&W #23; PR&NE #23 - #1272 - #23; To CNE (1st) #2 – (1st) #104 – (1st) #106 - #77)

CNE 46

To NH #3430 in Sep 1927. Condemned in Oct 1950. (CNE / NH Class Y-3)

CNE 47

Sold to Fitzhugh-Luther & Co. (locomotive broker) on 22 Apr 1905 for $1850.00 and by them, supposedly, to the Duluth, Rainy Lake & Winnipeg RR. Line became the Duluth, Winnipeg & Pacific RR in 1912. Final disposition unknown. (See HC RR note 4) (Class F-1) (Was HC #4; CNE&W #24; PR&NE #24 - #1273 - #24)

CNE 48

To NH #3431 in Sep 1927. Condemned in Jan 1949. (CNE / NH Class Y-3)

CNE 49

Was sold to the New Haven RR (their #257) in Nov 1904 for $6000.00 but returned to the CNE in Nov 1905 at a cost of $6018.72 and again renumbered to (1st) #25. Was condemned on 30 Apr 1915 for $350.00 scrap value. (See HC RR note 5) (Class F-1) (Was HC #5; CNE&W #25; PR&NE #25 - #1274 - #25; To CNE #75)

CNE 50

Became CNE (2nd) #47 in Aug 1922 on delivery of new 0-8-0 (3rd) #25. Condemned in July 1925. (See CNE&W note 12) (Class P-4) ( Was CNE&W (2nd) #11; PR&NE #11 - #1260 - #11; To CNE (2nd) #47)

CNE 51

To NH #3432 in Sep 1927. Condemned in Mar 1952. (CNE / NH Class Y-3)

CNE 52

Was sold to the New Haven RR (their #258) in Nov 1904 for $6000.00 but returned to the CNE in Nov 1905 at a cost of $6000.00 and again renumbered to (1st) #26. Was sold to E.H. Wilson Co. (locomotive broker) in Sep 1907 for $2000.00. Final disposition unknown. (See HC RR note 6) (Class F-1) (Was HC #6; CNE&W #26; PR&NE #26 - #1275 - #26)

CNE 53

Was condemned on 31 Dec 1915 for scrap value of $300.00. (See CNE&W note 14) (Class P-4) (Was CNE&W (2nd) #12; PR&NE #12 - #1261 - #12; To CNE (1st) #12)

CNE 54

To NH #3433 in Sep 1927. Condemned in May 1950. (CNE / NH Class Y-3)

CNE 55

Sold on 14 Sep 1906 to Fitzhugh-Luther & Co. (locomotive broker) for $1750.00 and by them to the Spokane & Inland Empire RR (their #6). Resold in 1911 to the Pacific & Eastern RR (their (2nd) #5) of Medford, OR. This line became the Brownlee Olds Lumber. Co. in 1921. Final disposition unknown. (See CNE&W note 32) (Class F-1) (Was CNE&W #27; PR&NE #27 - #1276 - #27)

CNE 56

Condemned on 30 Sep 1915 for $350.00 scrap value. (See CNE&W note 16) (Class P-4) (Was CNE&W (1st) #13; PR&NE #13 - #1262 - #13)

CNE 57

To NH #3434 in Sep 1927. Condemned in Mar 1952. (CNE / NH Class Y-3)

CNE 58

Was sold in May 1903 to Fitzhugh & Co. (locomotive broker) for $2800.00.Resold to the Weyerhauser Lumber Co. and used on their Hawthorne, Nebagamon & Superior RR (#2) in Wisc. Sold again in 1908 to the Mesabe Southern RR (#4) in Minn. And by them to the Duluth & Northeastern RR (#12) also in Minn. Was wrecked and scrapped in 1924 but it’s boiler was used in the D&NE roundhouse in Cloquet, Minn. for sometime after. (See CNE&W note 33) (Class F-1) (Was CNE&W #28; PR&NE #28 - #1277 - #28)

CNE 59

Became CNE (2nd) #48 in Aug 1922 on delivery of new 0-8-0 (3rd) #28. Condemned in Apr 1926. (See CNE&W note 17) (Class P-4) (Was CNE&W (2nd) #14; PR&NE #14 - #1263 - #14; To CNE (2nd) #48)

CNE 60

To NH #3415 in Sep 1927. Condemned in Mar 1952. (CNE / NH Class Y-3)

CNE 61

Sold in Nov 1902 to The Males Co. (locomotive broker) of New York City for $2300.00 and resold by them to the New York & Pennsylvania RR (their #29) in May 1903. Remained on the New York &Pennsylvania RR roster until at least 1929 and possibly later. (See CNE&W note 34) (Class F-1) (Was CNE&W #29; PR&NE #29 - #1278 - #29)

CNE 62

To NH #3416 in Sep 1927. Condemned in Mar 1952. (CNE / NH Class Y-3)

CNE 63

Sold in Nov 1902 to The Males Co. (locomotive broker) of New York City for $2300.00 and resold by them to the Detroit Southern RR in May or June 1903 for use on their Ohio Southern Division (#29). In May 1905 both these lines became the Detroit, Toledo & Ironton RR. Locomotive was scrapped about 1921. (See CNE&W note 35) (Class F-1) (Was CNE&W #30; PR&NE #30 - #1279 - #30)

CNE 64

To NH #3417 in Sep 1927. Condemned in Oct 1950. (CNE / NH Class Y-3)

CNE 65

Possible candidate for the #76 spot in the 1913 renumbering but no record found that it ever carried this number. Condemned on 20 Oct 1913 for scrap value of $323.00. (See CNE&W note 36) (Class F-1) (Was CNE&W #31; PR&NE #31)

CNE 66

Condemned on 31 Dec 1915 for scrap value of $300.00. (See HC note 2) (Class P-3) (Was HC #2; CNE&W (1st) #22 – (2nd) #19; PR&NE #19 - #1268 - #19; To CNE #223 – (2nd) #31)

CNE 67

To NH #3418 in Sep 1927. Condemned in Jan 1949. (CNE / NH Class Y-3)

CNE 68

Condemned on 31 Dec 1912 for scrap value of $500.00. (See CNE&W note 37) (Class F-1) (Was CNE&W #32; PR&NE #32 - #1281 - #32)

CNE 69

Condemned on 30 Apr 1915 for scrap value of $250.00. (See CNE&W note 24) (Class P-3) (Was CNE&W #20; PR&NE #20 - #1269 - #20; CNE (1st) #20 - #224)

CNE 70

To NH #3419 in Sep 1927. Condemned in Jun 1950. (CNE / NH Class Y-3)

CNE 71

Was sold in May 1903 to Fitzhugh & Co. (locomotive broker) for $2800.00 who in turn sold it to the Weed Lumber Co. in Weed, CA where it ran on their California Northeastern RR (locally called the “CNE”) as their #3. Became S.P. (Class C-6) (2nd) #2500 on 1 Sep 1910. Was condemned on 31 Jul 1927 and scrapped in Sacremento, CA on 23 Sep 1927. (See CNE&W note 38 (Class F-1) (Was CNE&W #33; PR&NE #33 - #1282 - #33)

CNE 72

Condemned on 30 Sep 1919 for scrap value of $400.00. (See HC RR note 1) (Class P-3) (Was HC #1; CNE&W #21; PR&NE #21 - #1270 - #21;  CNE (1st) #21 - #225)

CNE 73

Was wrecked at Canton, CT on 23 Jun 1899. Cylinders were simpled to 20” X 24” in the Hartford, CT shops in late 1903 or early 1904. Condemed on 1 July 1925. (See PR&NE note 38) (Class F-2) (Was PR&NE #34; To CNE #85)

CNE 74

Cylinders were simpled to 20” X 24” in the Hartford, CT shops in late 1903 or early 1904. Condemned in Oct 1925. (See PR&NE note 39) (Class F-2) (Was PR&NE #35; To CNE #86)

CNE 75

Was rebuilt with new BLW boiler in the Hartford, CT shops in July 1900 at a cost of $5231.25. Condemned on 20 Nov 1919. (See H&CW note 16) (Class P-2) (Was H&CW #16; CNE&W #16; PR&NE #16 - #1265 - #16; CNE (1st) #16 - #220)

CNE 76

Cylinders were simpled to 20” X 24” in the Hartford, CT shops in late 1904 or early 1905. Condemned in Jun 1925. (See PR&NE note 40) (Class F-2) (Was PR&NE #36; To CNE #87)

CNE 77

Was reboilered at the Hartford, CT shops in Apr 1901 at a cost of $5650.54. Condemned in Oct 1925. (See H&CW note 16) (Class P-2) (Was H&CW #17; CNE&W #17; PR&NE #17 - #1266 - #17; CNE (1st) #17 - #221)

CNE 78

Cylinders were simpled to 20” X 24” in the Hartford, CT shops in late 1904 or early 1905. Condemned in Jun 1925. (See PR&NE note 41) (Class F-2) (Was PR&NE #37; To CNE #88)

CNE 79

Was reboilered and fitted with extended saddle at the Hartford, CT shops in Jul 1901at a cost of $5061.02. Condemned in July 1926. See H&CW note 18) (Class P-2) (Was H&CW #18; CNE&W #18; PR&NE #18 - #1267 - #18; CNE (1st) #18 - #222)

CNE 80

Cylinders were simpled to 20” X 24” in the Hartford, CT shops in late 1904 or early 1905. Condemned in Jun 1926. (See PR&NE note 42) (Class F-2) (Was PR&NE #38; To CNE #89)

CNE 81

Condemned in Mar 1925. (See ND&C note 13) (Class P-2) (Was ND&C (2nd) #8; CNE #226)

CNE 82

Cylinders were simpled to 20” X 24” in the Hartford, CT shops in late 1904 or early 1905. Condemned on 18 July 1925. (See PR&NE note 43) (Class F-2) (Was PR&NE #39; To CNE #90)

CNE 83

Condemned in July 1926. (See ND&C note 15) (Class P-2) (Was ND&C (2nd) #9; CNE #227)

CNE 84

Was sold on 4 Sep 1905 to the Fitzhugh – Luther & Co. (locomotive broker) for $3000.00. Apparently ended up on a railroad somewhere in the California / Nevada desert area. Final disposition unknown. (See PR&NE note 44) (Class F-2) (Was PR&NE #40)

CNE 85

Condemned in July 1926. (See ND&C note 16) (Class P-2) (Was ND&C #10; CNE #228)

CNE 86

Condemned on 17 July 1925. (Class F-3) (To CNE (2nd) #100 on 16 Nov 1913)

CNE 87

Cylinders simpled to 18” X 24” and drivers extended to 69” during rebuilding in Hartford, CT shops in 1908. Condemned in Nov 1926. (See PR&NE note 6) (Class P-2) (Was PR&NE (2nd) #4; CNE (1st) #4 - #200 - #229)

CNE 88

Condemned in Oct 1926. (Class F-3) (To CNE (2nd) #101 on 16 Nov 1913)

CNE 89

Cylinders simpled to 18” X 24” and drivers extended to 69” during rebuilding in Hartford, CT shops in 1909. Condemned in Nov 1925. (See PR&NE note 12) (Class P-2) (Was PR&NE (2nd) #8; CNE #8 - #202 - #230)

CNE 90

Condemned in early 1925. (Class F-3) (To CNE (2nd) #102 on 16 Nov 1913)

CNE 91

Cylinders simpled to 18” X 24” and drivers extended to 69” during rebuilding in Hartford, CT shops in 1911. Condemned on 10 Oct 1926. (See PR&NE note 10) (Class P-2) (Was PR&NE (2nd) #7; CNE #7 - #201 - #231)\

CNE 92

Condemned in Oct 1926. (Class F-3) (To CNE (2nd) #103 on 16 Nov 1913)

CNE 93

Condemned in early 1925. (Class F-3) (To CNE (2nd) #104 on 16 Nov 1913)

CNE 94

Bought from the New Haven RR on 19 Aug 1914. Was NH #961 and #1504. Was renumbered back to NH #1504 in Sep 1927. Condemned at Midway, CT on 30 Sep 1927. (CNE Class P-2 / NH Class C-15)

CNE 95

Condemned in late 1925. (Class F-3) (To CNE (2nd) #105 on 16 Nov 1913)

CNE 96

Bought from the New Haven RR on 17 Oct 1914. Was NH #26 and #1506. Condemned in Dec 1926. (CNE Class P-2 / NH Class C-15)

CNE 97

Condemned in late 1925. (Class F-3) (To CNE (2nd) #106 on 16 Nov 1913)

CNE 98

Was renumbered in Aug 1922 on delivery of new 0-8-0 #25. Condemned in Jul 1925. (See CNE&W note 12) (Class P-4) (Was CNE&W (2nd) #11; PR&NE #11 - #1260 - #11; CNE (1st) #11 - (2nd) #25)

CNE 99

Condemned in Jun 1926. (Class F-3) (To CNE (2nd) #107 on 16 Nov 1913)

CNE 100

Was renumbered in Aug 1922 on delivery of new 0-8-0 #28. Condemned in Apr 1926. (See CNE&W note 17) (Class P-4) (Was CNE&W (2nd) #14; PR&NE #14 - #1263 - #14; CNE (1st) #14 - (2nd) #28)

CNE 101

Condemned in Dec 1926. (Class F-3) (To CNE (2nd) #108 on 16 Nov 1913)

CNE 102

Condemned in Dec 1925. (Class F-3) (To CNE (2nd) #109 on 16 Nov 1913)

CNE 103

Condemned in Mar 1928. (CNE / NH Class P-1) (Was CNE #300; To NH #50 in Jun  1927)

CNE 104

Became CNE #110 on 16 Nov 1913. Transferred to the New Haven RR in 1926 as their #110. Condemned in Aug 1927. (CNE / NH Class F-3)

CNE 105

Condemned at Midway, CT on 29 Feb 1928. (CNE / NH Class P-1) (Was CNE #301; To NH #51 in Jun 1927)

CNE 106

Condemned in Nov 1925. (Class F-3) (To CNE #111 on 16 Nov 1913)

CNE 107

Condemned on 31 Dec 1935. Scrapped at Readville, MA on 4 Jun 1936. (CNE / NH Class P-1) (Was CNE #302; To NH #52in Jun 1927)

CNE 108

Condemned in late 1925. (Class F-3) (To CNE #112 on 16 Nov 1913)

CNE 109

Condemned in Dec 1926. (Class F-3) (To CNE #113 on 16 Nov 1913)

CNE 110

Condemned in late 1925. (Class F-3) (To CNE #114 on 16 Nov 1913)

CNE 111

Cost $14,844.00 new. Condemned in late 1925. (Class F-3) (To CNE #115 on 16 Nov 1913)

CNE 112

Cost $14,844.00 new. Condemned in Jun 1926. (Class F-3) (To CNE #116 on 16 Nov 1913)

CNE 113

Cost $14,844.00 new. To CNE #117 on 16 Nov 1913. Transferred to the New Haven RR in 1926 as their #117. Condemned in Mar 1928. (Class F-3)

CNE 114

Cost $14,844.00 new. To CNE #118 on 16 Nov 1913. Transferred to the New Haven RR in 1926 as their #118. Condemned in Mar 1928. (Class F-3)

CNE 115

Cost $14,844.00 new. Condemned on 16 July 1925. (Class F-3) (To CNE #119 on 16 Nov 1913)

CNE 116

Cost $14,844.00 new. Condemned in late 1925. (Class F-3) (To CNE #120 on 16 Nov 1913)

CNE 117

Condemned as of 30 Apr 1915 for scrap value of $350.00. (See HC RR note 5) (CNE Class F-1) / NH Class P-4-a) (Was HC #5; CNE&W #25; PR&NE #25 - #1274 - #25; CNE (1st) #25; NH #257; CNE (1st) #25)

CNE 118

No proof as yet found that this number was ever used. However, this particular locomotive was condemned as of 20 Oct 1913 and written off for $350.00 scrap value. (See CNE&W note 36) (Class F-1) (Was CNE&W #31; PR&NE #31; CNE (1st) #31)

CNE 119

Was the Hartford, CT yard switcher for many years. Condemned on 1 Jul 1914 for $350.00 scrap value. (See HC RR note 3) (Class Y-1) (Was HC #3; CNE&W #23; PR&NE #23 - #1272 - #23; CNE (1st) #23 – (1st) #2 – (1st) #104 – (1st) #106)

CNE 120

Was wrecked at Canton, CT on 23 Jun 1899. Cylinders were simpled to 20” X 24” in late 1904 or early 1905. Condemned on 1 July 1925. (See PR&NE note 38) (Class F-2) (Was PR&NE #34; To CNE #34)

CNE 121

Cylinders were simpled to 20” X 24” in late 1904 or early 1905. Condemned in Oct 1925. (See PR&NE note 39) (Class F-2) (Was PR&NE #35; To CNE (1st) #35)

CNE 122

Cylinders were simpled to 20” X 24” in late 1904 or early 1905. Condemned in Jun 1925. (See PR&NE note 40) (Class F-2) (Was PR&NE #36; To CNE (1st) #36)

CNE 123

Cylinders were simpled to 20” X 24” in late 1904 or early 1905. Condemned in Jun 1925. (See PR&NE note 41) (Class F-2) (Was PR&NE #37; To CNE (1st) #37)

CNE 124

Cylinders were simpled to 20” X 24” in late 1904 or early 1905. Condemned in Jun 1926. (See PR&NE note 42) (Class F-2) (Was PR&NE #38; To CNE (1st) #38)

CNE 125

Cylinders were simpled to 20” X 24” in late 1904 or early 1905. Condemned on 18 July 1925. (See PR&NE note 43) (Class F-2) (Was PR&NE #39; To CNE (1st) #39)

CNE 126

Sold to the New Haven RR in Nov 1904 for $10,250.00 becoming their #2400. Sold back to the CNE in Nov 1905 for $9,367.34 becoming (1st) #100 again. Became (3rd) #1 on 16 Nov 1913 and (2nd) #7 in Aug 1925. Could have been numbered NH #2350 except was condemned in Sep 1927. (CNE Class Y-1 / NH Class T-3)

CNE 127

Condemned on 17 July 1925. (Class F-3) (Was CNE (1st) #41)

CNE 128

Sold to the New Haven RR in Nov 1904 for $10,250.00 becoming their #2401. Sold back to the CNE in Nov 1905 for $9,341.14 becoming (1st) #101 again. Became (2nd) #2 on 16 Nov 1913 and New Haven #2351 in Sep 1927. Was condemned on 31 Jan 1935 and was scrapped at Readville, MA on 28 Mar 1935. (CNE Class Y-1 / NH Class T-3)

CNE 129

Condemned in Oct 1926. (Class F-3) (Was CNE (1st) #42)

CNE 130

Sold on 21 May 1930 to construction company Lathrop & Shea (their #510). Final disposition not known. (CNE Class Y-1 / NH Class T-3)

(To CNE (2nd) #3 on 16 Nov 1913; To NH #2352 in Sep 1927)

CNE 131

Condemned in early 1925. (Class F-3) (Was CNE (1st) #43)

CNE 132

Rebuilt to an 0-6-0T by the New Haven RR in 1938. Was condemned in May 1953 and was apparently the last CNE locomotive to exist. (CNE Class Y-1 / NH Class T-3) (To CNE (3rd) #4 on 16 Nov 1913; To NH #2353 in Sep 1927)

CNE 133

Condemned in Oct 1926. (Class F-3) (Was CNE (1st) #44)

CNE 134

Became (1st) #106 when new 0-6-0 (2nd) #104 was delivered in Jul 1910. Condemned on 1 July 1914 for $350.00 scrap value. (See HC RR note 3) (Class Y-1) (Was HC#3; CNE&W #23; PR&NE #23 - #1272 - #23; CNE (1st) #23 – (1st) #2; To CNE (1st) #106 - #77)

CNE 135

Became (3rd) #5 on 16 Nov 1913. To New Haven RR #2354 in Sep 1927. Condemned on 8 Mar 1930. (CNE Class Y-1 / NH Class T-3)

CNE 136

Condemned in early 1925. (Class F-3) (Was CNE (1st) #45)

CNE 137

Became (2nd) #6 on 16 Nov 1913. To New Haven RR #2355 in Sep 1927. Condemned in Dec 1935. (CNE Class Y-1 / NH Class T-3)

CNE 138

Condemned in late 1925. (Class F-3) (Was CNE (1st) #46)

CNE 139

Became CNE #77 on 16 Nov 1913. Condemned on 1 July 1914 for $350.00 scrap value. (See HC RR note 3) (Class Y-1) (Was HC#3; CNE&W #23; PR&NE #23 - #1272 - #23; CNE (1st) #23 – (1st) #2 – (1st) #104; To CNE #77)

CNE 140

Condemned in late 1925. (Class F-3) (Was CNE (1st) #47)

CNE 141

Cost new $18,020.25. Originally assigned number changed to (2nd) #10 on 16 Nov 1913. To New Haven RR #10 in Oct 1927. Condemned in Dec 1946. (CNE /NH Class Y-2)

CNE 142

Condemned in Jun 1926. (Class F-3) (Was CNE (1st) #48)

CNE 143

Cost new $18,020.25. Originally assigned number changed to (2nd) #11 on 16 Nov 1913. To New Haven RR #11 in Oct 1927. Condemned in Jan 1946. (CNE /NH Class Y-2)

CNE 144

Condemned in Dec 1926. (Class F-3) (Was CNE #49)

CNE 145

Cost new $18,020.25. Originally assigned number changed to (2nd) #12 on 16 Nov 1913. To New Haven RR #12 in Oct 1927. Condemned in July 1948. (CNE /NH Class Y-2)

CNE 146

Was the locomotive E/B Train #194 involved in the head-on wreck at New Hartford, CT on 22 Dec 1916. Condemned in Dec 1925. (Class F-3) (Was CNE (1st) #50)

CNE 147

Transferred to New Haven RR #110 in late 1926. Condemned in Aug 1927. (CNE / NH Class F-3) (Was CNE (1st) #51)

CNE 148

Condemned in Nov 1925. (Class F-3) (Was CNE (1st) #52)

CNE 149

Condemned in late 1925. (Class F-3) (Was CNE (1st) #53)

CNE 150

Condemned in Dec 1926. (Class F-3) (Was CNE (1st) #54)

CNE 151

Condemned in late 1925. (Class F-3) (Was CNE (1st) #55)

CNE 152

Condemned in late 1925. (Class F-3) (Was CNE (1st) #56)

CNE 153

Condemned in Jun 1926. (Class F-3) (Was CNE (1st) #57)

CNE 154

Transferred to the New Haven RR #117 in late 1926. Condemned in Mar 1928. (CNE / NH Class F-3) (Was CNE (1st) #58)

CNE 155

Transferred to the New Haven RR #118 in late 1926. Condemned in Mar 1928. (CNE / NH Class F-3) (Was CNE (1st) #59)

CNE 156

Condemned on 16 July 1925. (Class F-3) (Was CNE (1st) #60)

CNE 157

Condemned in late 1925. (Class F-3) (Was CNE (1st) #61)

CNE 158

Was one of the locomotives involved in the head-on wreck of the night freights in New Hartford, CT on 22 Dec 1916. Condemned at Midway, CT in Sep 1929. (CNE Class F-4 / NH Class K-6) (Was CNE #400; To NH #550 in Sep 1927)

CNE 159

Was one of the locomotives involved in the head-on wreck of the night freights in New Hartford, CT on 22 Dec 1916. Condemned on 31 Dec 1935 and was scrapped at Readville, MA on 12 Aug 1936. (CNE Class F-4 / NH Class K-6) (Was CNE #401; To NH #551 in Sep 1927)

CNE 160

Condemned on 31 Mar 1935. Scrapped at the Readville, MA shops on 22 Aug 1935. (CNE Class F-4 / NH Class K-6) (Was CNE #402; To NH #552 in Sep 1927)

CNE 161

Condemned on 31 Mar 1935. Scrapped at the Readville, MA shops on 23 Aug 1935. (CNE Class F-4 / NH Class K-6) (Was CNE #403; To NH #553 in Sep 1927)

CNE 162

Condemned on 31 Mar 1935. Scrapped at the Readville, MA shops on 27 Aug 1935. (CNE Class F-4 / NH Class K-6) (Was CNE #404; To NH #554 in Sep 1927)

CNE 163

Condemned on 31 Mar 1935. Scrapped at the Readville, MA shops on 22 Aug 1935. (CNE Class F-4 / NH Class K-6) (Was CNE #405; To NH #555 in Sep 1927)

CNE 164

Sold by the New Haven RR to the CNE on 8 Oct 1914. Condemned in Aug 1927. (CNE / NH Class K-2) (Was NH #265 to 1904 and NH #545 to 1914)

CNE 165

Condemned on 28 Oct 1950. (CNE / NH Class F-5) (Was CNE #500 to 16 Nov 1913 only; To NH #150 in Sep 1927)

CNE 166

Condemned in Jun 1950. (CNE / NH Class F-5) (Was CNE #501 to 16 Nov 1913 only; To NH #151 in Sep 1927)

CNE 167

Condemned in Jan 1950. (CNE / NH Class F-5) (Was CNE #502 to 16 Nov 1913 only; To NH #152 in Sep 1927)

CNE 168

Condemned in Jun 1950. (CNE / NH Class F-5) (Was CNE #503 to 16 Nov 1913 only; To NH #153 in Sep 1927)

CNE 169

Condemned in Mar 1949. (CNE / NH Class F-5) (Was CNE #504 to 16 Nov 1913 only; To NH #154 in Sep 1927)

CNE 170

Condemned in Oct 1946. (CNE / NH Class F-5) (Was CNE #505 to 16 Nov 1913 only; To NH #155 in Sep 1927)

CNE 171

Condemned in Oct 1946. (CNE / NH Class F-5) (Was CNE #506 to 16 Nov 1913 only; To NH #156 in Sep 1927)

CNE 172

Condemned in Jun 1946. (CNE / NH Class F-5) (Was CNE #507 to 16 Nov 1913 only; To NH #157 in Sep 1927)

CNE 173

Condemned in July 1948. (CNE / NH Class F-5) (Was CNE #508 to 16 Nov 1913 only; To NH #158 in Sep 1927)

CNE 174

Condemned in Jan 1946. (CNE / NH Class F-5) (Was CNE #509 to 16 Nov 1913 only; To NH #159 in Sep 1927)

CNE 175

Condemned in Jan 1946. (CNE / NH Class F-5) (Was CNE #510 to 16 Nov 1913 only; To NH #160 in Sep 1927)

CNE 176

Condemned in July 1950. (CNE / NH Class F-5) (Was CNE #511 to 16 Nov 1913 only; To NH #161 in Sep 1927)

CNE 177

Condemned in Jan 1946. (CNE / NH Class F-5) (Was CNE #512 to 16 Nov 1913 only; To NH #162 in Sep 1927)

CNE 178

Condemned in July 1948. (CNE / NH Class F-5) (Was CNE #513 to 16 Nov 1913 only; To NH #163 in Sep 1927)

CNE 179

Condemned on 27 Apr 1949. (CNE / NH Class F-5) (Was CNE #514 to 16 Nov 1913 only; To NH #164 in Sep 1927)

CNE 180

Condemned in Jan 1946. (CNE / NH Class J-2) (To NH #3105 in Sep 1927)

CNE 181

Condemned in Jan 1946. (CNE / NH Class J-2) (To NH #3106 in Sep 1927)

CNE 182

Condemned in Jan 1946. (CNE / NH Class J-2) (To NH #3107 in Sep 1927)

CNE 183

Was renumbered about 1910 after most other 4-4-0’s below #220 were scrapped. Condemned in Nov 1926. (See PR&NE note 6) (Class P-2) (Was PR&NE (2nd) #4; CNE (1st) #4; To CNE #229 – (2nd) #41)

CNE 184

Was renumbered about 1910 after most other 4-4-0’s below #220 were scrapped. Condemned on 10 Oct 1926. (See PR&NE note 10) (Class P-2) (Was PR&NE (2nd) #7; CNE (1st) #7; To CNE #231 – (2nd) #43)

CNE 185

Was renumbered about 1910 after most other 4-4-0’s below #220 were scrapped. Condemned in Nov 1925. (See PR&NE note 12) (Class P-2) (Was PR&NE (2nd) #8; CNE #8; To CNE #230 – (2nd) #42)

CNE 186

Was a possible candidate for the missing #34 spot in the 16 Nov 1913 renumbering. Was condemned on 20 Oct 1913 for scrap value of $350.00. (See P&E Ry note 2) (Class P-3) (Was P&E Ry (2nd) #1)

CNE 187

Condemned as of 31 Dec 1912 for a value of $500.00 and probable scrapped soon after. (See P&E Ry note 4) (Class P-2) (Was P&E Ry (2nd) #2)

CNE 188

Condemned on 29 Sep 1909. (See P&E Ry note 14) (Was P&E Ry (2nd) #6)

CNE 189

Condemned and sold 7 Nov 1908 to A.B. Ehrlich Co. (scrap dealer) for $355.63. (See P&E Ry note 15) (Was P&E Ry #7)

CNE 190

Condemned in Apr 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $200.00. (See P&E Ry note 6) (Was P&E Ry (2nd) #3)

CNE 191

Sold on 2 Nov 1906 to an U.I. Harris (scrap dealer) of Hartford, CT for $625.00. (See CW note 11) (Was CW #10; H&CW #10; CNE&W #10; PR&NE #10; CNE (1st) #10)

CNE 192

Condemned in Apr 1908 and sold to Sinsman & Blumenthal Co. (scrap dealer) for $200.00. (See H&CW note 15) (Was H&CW #15; CNE&W #15; PR&NE #15 - #1264 - #15; CNE (1st) #15)

CNE 193

Condemned in Jun 1907 for value of $562.50 and scrapped soon after. (See ND&C RR note 5) (Was D&C #2 “Bangall”; NYB&M #14; ND&C #2)

CNE 194

Sold to Sinsman & Fisher Co. (scrap dealer) in Aug 1906 for $525.00. (See ND&C RR note 8) (Was D&C #4 “Lagrange”; NYB&M #11; ND&C #4)

CNE 195

Condemned on 9 Sep 1909. (See ND&C RR note 9) (Was NYB&M #2; ND&C #5)

CNE 196

Condemned on 31 Dec 1912 for value of $500.00 and sold soon after for scrap. (See ND&C RR note 10) (Was NYB&M #6; ND&C #6)

CNE 197

Condemned on 9 Sep 1909. (See ND&C RR note 11) (Was NYB&M #1; ND&C #7)

CNE 198

Condemned on 20 Nov 1919. (See H&CW note 16) (Was H&CW #16; CNE&W #16; PR&NE #16 - #1265 - #16; CNE (1st) #16; To CNE (2nd) #35)

CNE 199

Condemned in Oct 1925. (See H&CW note 17) (Class P-2) (Was H&CW #17; CNE&W #17; PR&NE #17 - #1266 - #17; CNE (1st) #17; To CNE (2nd) #36)

CNE 200

Condemned in July 1926. (See H&CW note 18) (Class P-2) (Was H&CW #18; CNE&W #18; PR&NE #18 - #1267 - #18; CNE (1st) #18; To CNE (2nd) #37)

CNE 201

Condemned 31 Dec 1915 for $300.00 scrap value. (See HC RR note 2) (Class P-3) (Was HC #2; CNE&W #22 – (2nd) #19; PR&NE #19 - #1268 - #19; CNE (1st) #19; To CNE (2nd) #31)

CNE 202

Condemned 30 Apr 1915 for $250.00 scrap value. (See CNE&W note 24) (Class P-3) (Was CNE&W #20; PR&NE #20 - #1269 - #20; CNE (1st) #20; To CNE (2nd) #32)

CNE 203

Condemned 30 Sep 1919 for $400.00 scrap value. (See HC RR note 1) (Class P-3) (Was HC #1; CNE&W #21; PR&NE #21 - #1270 - #21; CNE (1st) #21; To CNE (2nd) #33)

CNE 204

Condemned in Mar 1925. (See ND&C RR note 13) (Class P-2) (Was ND&C (2nd) #8; To CNE (2nd) #38)

CNE 205

Condemned in July 1926. (See ND&C RR note 15) (Class P-2) (Was ND&C (2nd) #9; To CNE (2nd) #39)

CNE 206

Condemned in July 1926. (See ND&C RR note 16) (Class P-2) (Was ND&C (2nd) #10; To CNE (2nd) #40)

CNE 207

Was renumbered about 1910 to this number from #200 after all other 4-4-0’s below #220 were scrapped. Condemned in Nov 1926. (See PR&NE note 6) (Class P-2) (Was PR&NE (2nd) #4; CNE (1st) #4 - #200; To CNE (2nd) #41)

CNE 208

Was renumbered about 1910 to this number from #202 after all other 4-4-0’s below #220 were scrapped. Condemned in Nov 1925. (See PR&NE note 12) (Class P-2) (Was PR&NE (2nd) #8; CNE #8 - #202; To CNE (2nd) #42)

CNE 209

Was renumbered about 1910 to this number from #201 after all other 4-4-0’s below #220 were scrapped. Condemned on 10 Oct 1926. (See PR&NE note 10) (Class P-2) (Was PR&NE (2nd) #7; CNE (1st) #7 - #201; To CNE (2nd) #43)

CNE 210

Condemned in Mar 1928. (CNE / NH Class P-1) (To CNE (2nd) #50; To NH #50 after Jul 1927)

CNE 211

Condemned in Feb or Mar 1928. (CNE / NH Class P-1) (To CNE (2nd) #51; To NH #51 after Jul 1927)

CNE 212

Condemned in Dec 1935. (CNE / NH Class P-1) (To CNE (2nd) #52; To NH #52 after Jul 1927)

CNE 213

Cost new $16,560.00. Condemned in Sep 1929. (CNE Class F-4 / NH Class K-6) (To CNE #125; To NH #550 in Sep 1927)

CNE 214

Cost new $16,560.00. Condemned in Dec 1935. (CNE Class F-4 / NH Class K-6) (To CNE #126; To NH #551 in Sep 1927)

CNE 215

Cost new $16,560.00. Condemned on 31 Mar 1935. (CNE Class F-4 / NH Class K-6) (To CNE #127; To NH #552 in Sep 1927)

CNE 216

Cost new $16,560.00. Condemned on 31 Mar 1935. (CNE Class F-4 / NH Class K-6) (To CNE #128; To NH #553 in Sep 1927)

CNE 217

Cost new $16,560.00. Condemned in Dec 1935. (CNE Class F-4 / NH Class K-6) (To CNE #129; To NH #554 in Sep 1927)

CNE 218

Cost new $16,560.00. Condemned on 31 Mar 1935. (CNE Class F-4 / NH Class K-6) (To CNE #130; To NH #555 in Sep 1927)

CNE 219

Cost new $20,596.46. Condemned on 28 Oct 1950. (CNE / NH Class F-5) (To CNE #150 on 16 Nov 1913; To NH #150 in Sep 1927)

CNE 220

Cost new $20,596.46. Condemned in Jun 1950. (CNE / NH Class F-5) (To CNE #151 on 16 Nov 1913; To NH #151 in Sep 1927)

CNE 221

Cost new $20,596.46. Condemned in Jan 1946. (CNE / NH Class F-5) (To CNE #152 on 16 Nov 1913; To NH #152 in Sep 1927)

CNE 222

Cost new $20,596.46. Condemned in Jun 1950. (CNE / NH Class F-5) (To CNE #153 on 16 Nov 1913; To NH #153 in Sep 1927)

CNE 223

Cost new $20,596.46. Condemned in Mar 1949. (CNE / NH Class F-5) (To CNE #154 on 16 Nov 1913; To NH #154 in Sep 1927)

CNE 224

Cost new $20,596.46. Condemned in Oct 1946. (CNE / NH Class F-5) (To CNE #155 on 16 Nov 1913; To NH #155 in Sep 1927)

CNE 225

Cost new $20,596.46. Condemned in Oct 1946. (CNE / NH Class F-5) (To CNE #156 on 16 Nov 1913; To NH #156 in Sep 1927)

CNE 226

Cost new $20,596.46. Condemned in Jun 1946. (CNE / NH Class F-5) (To CNE #157 on 16 Nov 1913; To NH #157 in Sep 1927)

CNE 227

Cost new $20,596.46. Condemned in Jul 1948. (CNE / NH Class F-5) (To CNE #158 on 16 Nov 1913; To NH #158 in Sep 1927)

CNE 228

Cost new $20,596.46. Condemned in Jan 1946. (CNE / NH Class F-5) (To CNE #159 on 16 Nov 1913; To NH #159 in Sep 1927)

CNE 229

Cost new $20,596.46. Condemned in Jan 1946. (CNE / NH Class F-5) (To CNE #160 on 16 Nov 1913; To NH #160 in Sep 1927)

CNE 230

Cost new $20,596.46. Condemned in July 1950. (CNE / NH Class F-5) (To CNE #161 on 16 Nov 1913; To NH #161 in Sep 1927)

CNE 231

Cost new $20,596.46. Condemned in Jan 1946. (CNE / NH Class F-5) (To CNE #162 on 16 Nov 1913; To NH #162 in Sep 1927)

CNE 232

Cost new $20,596.46. Condemned in July 1948. (CNE / NH Class F-5) (To CNE #163 on 16 Nov 1913; To NH #163 in Sep 1927)

CNE 233

Cost new $20,596.46. Condemned on 27 Apr 1949. (CNE / NH Class F-5) (To CNE #164 on 16 Nov 1913; To NH #164 in Sep 1927)